# Part 1: Theory

# 1. What is a Power Supply System?

A Power Supply System provides electrical power to a computer and its components.

Electricity from the wall socket cannot be used directly by computer components because:

- Wall power is AC (Alternating Current).

- Computer components need DC (Direct Current).

Therefore, a device is needed to convert AC to DC.

This device is called an SMPS.

# 2. What is SMPS?

SMPS = Switched Mode Power Supply

It is a hardware component inside the computer cabinet that converts AC power into different DC voltages required by computer components.

### Functions of SMPS

- Converts AC to DC.

- Supplies power to motherboard.

- Supplies power to CPU.

- Supplies power to hard disk/SSD.

- Supplies power to graphics card.

- Protects components from voltage fluctuations.

### Common Output Voltages

| Voltage | Usage |
| --- | --- |
| +12V | CPU, Fans, Graphics Card |
| +5V | USB Devices, Hard Disk |
| +3.3V | RAM, Motherboard Components |

## Real-Life Example

Think of SMPS as a translator.

Wall Socket → AC Power

Computer Components → Need DC Power

SMPS translates AC into DC so components can understand and use it.

# 3. Parts of an SMPS

### Main Parts

- Cooling Fan

  - Prevents overheating.

- Power Switch

  - Turns power supply ON/OFF.

- Power Socket

  - Connects AC cable.

- Output Connectors

  - Connect motherboard and storage devices.

# 4. Advantages of SMPS

- High efficiency

- Less heat generation

- Compact size

- Lightweight

- Reliable operation

# 5. What is UPS?

UPS = Uninterruptible Power Supply

A UPS provides backup power when electricity fails.

### Functions of UPS

- Provides temporary backup power.

- Protects against power cuts.

- Protects against voltage fluctuations.

- Allows safe shutdown of computer.

## Real-Life Example

Imagine you are saving an important project.

Suddenly electricity goes off.

Without UPS:

- Computer shuts down immediately.

- Unsaved work may be lost.

With UPS:

- Computer continues running for a few minutes.

- You can save your work safely.

# 6. Types of UPS

### 1. Offline UPS

- Most common.

- Used in homes and small offices.

### 2. Online UPS

- Provides continuous power.

- Used in servers and data centers.

### 3. Line Interactive UPS

- Better voltage regulation.

- Used in offices.

# 7. SMPS vs UPS

| Feature | SMPS | UPS |
| --- | --- | --- |
| Full Form | Switched Mode Power Supply | Uninterruptible Power Supply |
| Purpose | Converts AC to DC | Provides Backup Power |
| Installed | Inside Computer | Outside Computer |
| Backup Battery | No | Yes |
| Protects From | Voltage Issues | Power Failure |

# Part 2: Practical

# Practical 1: Identify Power Cables

Ask students to identify the following cables.

## A. AC Power Cable

Used to connect:

- UPS to Wall Socket

- Computer to UPS

Identification:

- Thick cable

- Three-pin connector

## B. 24-Pin ATX Connector

Used to provide power to:

- Motherboard

Identification:

- Largest connector from SMPS.

## C. SATA Power Connector

Used for:

- SSD

- HDD

- DVD Drive

Identification:

- Flat black connector.

## D. CPU Power Connector (4/8 Pin)

Used to power:

- Processor

Connected near CPU socket on motherboard.

# Practical 2: Locate Power Components

Students should open a CPU cabinet and identify:

✓ SMPS

✓ Motherboard Power Connector

✓ CPU Power Connector

✓ SATA Power Connector

✓ Cooling Fan

✓ Power Switch

# Practical 3: Check UPS Backup

### Steps

- Turn ON computer.

- Connect through UPS.

- Remove UPS input power.

- Observe system remains ON.

Discussion:

- How long does UPS provide backup?

- Why is UPS important?

## Practical 4: Check Battery Status Using CMD

Open Command Prompt:

powercfg /batteryreport

Generates battery report (for laptops).

Report location:

C:\Users\Username\battery-report.html

Students can open the report in browser.

## Practical 5: View Power Configuration

powercfg /a

Shows available sleep and power states.

## Practical 6: See System Uptime

systeminfo | find "Boot Time"

OR

net statistics workstation

Shows when the system was last started.

## Practical 7: Shutdown Timer

shutdown /s /t 60

Computer will shut down after 60 seconds.

Cancel shutdown:

shutdown /a

Students usually find this practical interesting.

## Practical 8: Restart Computer After 30 Seconds

shutdown /r /t 30

Explanation:

- /r = Restart

- /t = Time in seconds

Cancel:

shutdown /a

# Find Biggest Files on PC

File Explorer Search:

size:>100MB

OR

size:>1GB

Students can identify storage-hungry files.

# View Hidden Files

File Explorer → View → Hidden Items

Show:

- AppData folder

- Hidden system files

Explain why malware often hides here.

Open Notepad.

Type:

.LOG

Save file.

Close and reopen it.

# Personal Message Box

Create a file:

message.vbs

Paste:

MsgBox "Welcome to Today's Practical Session!"

Save and double-click.

A popup message appears.

Students can customize their own messages.

# Make the Computer Speak

Create:

speak.vbs

Paste:

Set speech=CreateObject("sapi.spvoice")
speech.Speak "Hello Students, Welcome to Computer Lab"

Save and run.

Computer will speak the message.

# Generate Random Numbers

CMD:

echo %random%

Run multiple times.

Create a simple guessing game.

Example:

echo Guess number between 1 and 100
echo Actual Number = %random%

# Digital Clock in CMD

Create:

@echo off
:time
cls
echo %time%
timeout /t 1 >nul
goto time

# Find All IP Addresses in Network

CMD:

arp -a

Shows devices recently connected on the network.

Students get excited seeing multiple IP addresses.

# Funny Folder Names

Create folders named:

Homework
Exams
Secret Files

Then use:

attrib +h Homework

Folder disappears.

Show hidden items to bring it back.

Useful introduction to file attributes.

## Folder Attributes Practical (Windows CMD)

### Step 1: Create Folders

Open Command Prompt (CMD) and run:

mkdir Homework

mkdir Exams

mkdir "Secret Files"

Or create all folders at once:

mkdir Homework Exams "Secret Files"

### Step 2: Hide the Homework Folder

attrib +h Homework

+h = Hidden Attribute

The Homework folder will no longer be visible in File Explorer.

### Step 3: Show Hidden Items

- Open File Explorer.

- Click on the View menu.

- Select Show → Hidden Items.

The Homework folder will become visible again.

### Step 4: Unhide the Folder

Run the following command in CMD:

attrib -h Homework

-h = Remove Hidden Attribute

## Extra Practical Activity

Make the folder Hidden + System:

attrib +h +s Homework

Now the folder may remain hidden even when Hidden Items is enabled.

To restore it:

attrib -h -s Homework

## Check Folder Attributes

attrib Homework

Example Output:

SH     Homework

### Attribute Meanings

| Attribute | Meaning |
| --- | --- |
| S | System |
| H | Hidden |
| R | Read Only |
| A | Archive |

# Create Password Protected ZIP

Right-click folder → Send To → Compressed Folder.

Then use WinRAR/7-Zip to add password.

Teach basic file security.

Encrypt Drive

Encrypt pendrive

Encrypt document

## Practical: Password Protect a Microsoft Word Document

### Steps

- Open Microsoft Word.

- Create a new document and add some text.

- Click File (or Office Button in Word 2007).

- Select Info.

- Click Protect Document.

- Choose Encrypt with Password.

- Enter a password.

- Re-enter the password to confirm.

- Save the document.

### Test

- Close the document.

- Open it again.

- Word will ask for the password before opening the file.

### Remove Password

- Open the document using the password.

- Go to File → Info → Protect Document → Encrypt with Password.

- Delete the existing password.

- Click OK and save the file.

## Practical: Password Protect a Microsoft Excel Workbook

### Steps

- Open Microsoft Excel.

- Create a workbook and enter some data.

- Click File.

- Select Info.

- Click Protect Workbook.

- Choose Encrypt with Password.

- Enter a password.

- Re-enter the password to confirm.

- Save the workbook.

### Test

- Close the workbook.

- Open it again.

- Excel will ask for the password before opening the file.

### Remove Password

- Open the workbook using the password.

- Go to File → Info → Protect Workbook → Encrypt with Password.

- Remove the existing password.

- Click OK and save the workbook.

# Extra Questions

- What is the full form of SMPS?

- Why is SMPS required in a computer?

- What is the full form of UPS?

- What is the purpose of UPS?

- What is the difference between AC and DC?

- Which connector powers the motherboard?

- Which connector powers the CPU?

- What is the use of a SATA power connector?

- What happens if there is no UPS during a power failure?

- Which command shows battery information in Windows?

### Conclusion

- SMPS converts AC power into usable DC power for computer components.

- UPS provides backup power during electricity failures.

- Students should be able to identify major power cables and connectors and understand their functions.