## Day 2 & Day 3 – Practical Networking Notes (Commands + Troubleshooting + Real-World Tips)

If you are preparing for a Networking Interview, Lab Practical, or Networking Job, theory alone is not enough. You should also know practical commands, troubleshooting methods, and real-world tips. Below are some important command-based notes that will help you in practical exams and interviews.

# Day 2 – Routers & Switches (Practical + Commands)

## 1. Check Network Adapter

### Command

ipconfig

### Purpose

- View the current IP Address.

- View the Default Gateway.

- Check whether the computer has received an IP address from DHCP.

### Example

IPv4 Address . . . . : 192.168.1.100

Subnet Mask . . . . : 255.255.255.0

Default Gateway . . : 192.168.1.1

## 2. View Detailed Network Information

### Command

ipconfig /all

### Displays

- MAC Address

- DHCP Status

- DNS Server

- Lease Time

- Default Gateway

### Interview Question

What is the difference between ipconfig and ipconfig /all?

Answer

- ipconfig displays basic network information.

- ipconfig /all displays detailed network information.

## 3. Refresh the IP Address

Sometimes the computer receives an incorrect IP address.

### Release the Current IP

ipconfig /release

### Request a New IP

ipconfig /renew

### Use When

- No Internet connection

- Incorrect IP Address

- DHCP-related issues

## 4. Clear the DNS Cache

### Command

ipconfig /flushdns

### Use

Clears old DNS records stored on the computer.

Useful when websites are not opening because of outdated DNS cache.

## 5. Test the Router

### Command

ping 192.168.1.1

### Successful Output

Reply from 192.168.1.1

### Failed Output

Request Timed Out

### Meaning

The computer cannot communicate with the router.

## 6. Test Internet Connectivity

### Command

ping 8.8.8.8

Google Public DNS Server

### If Successful

The Internet connection is working.

### If Failed

There may be a problem with the ISP or the router.

## 7. Test DNS Resolution

### Command

ping google.com

If:

- ping 8.8.8.8 works

- but ping google.com fails

Then the problem is related to the DNS server.

## 8. Check Physical Network Connection

### Command

netsh interface show interface

### Displays

- Connected

- Disconnected

- Disabled

## 9. View MAC Address

### Command

getmac

or

ipconfig /all

### Example

40-8D-5C-7B-11-22

A switch uses the MAC Address to forward data.

## 10. View the ARP Table

### Command

arp -a

### Displays

IP Address ↔ MAC Address Mapping

### Example

192.168.1.1

00-11-22-AA-BB-CC

### Interview Question

What is ARP?

Answer

ARP (Address Resolution Protocol) converts an IP Address into a MAC Address.

# Switch Practical

Suppose you have the following setup:

PC1

│

Switch

│

PC2

Copy a file from one computer to another.

Observe the Activity LED on the switch.

If the LED blinks, it means data is being transferred successfully.

## Open Network Connections

### Command

ncpa.cpl

This command opens the Network Connections window.

## Open Command Prompt Quickly

Press

Win + R

Type

cmd

Press Enter.

# Router Practical

Common Default Gateway Addresses

192.168.1.1

192.168.0.1

192.168.10.1

Open a web browser and enter

http://192.168.1.1

The router login page should open.

# Day 2 Interview Questions

### Router works on which OSI Layer?

Layer 3 (Network Layer)

### Switch works on which OSI Layer?

Layer 2 (Data Link Layer)

### Router uses which address?

IP Address

### Switch uses which address?

MAC Address

### What does a Router connect?

Different Networks

### What does a Switch connect?

Devices within the same network.

### Which cable connects a Router to a Switch?

Ethernet Cable (RJ45)

### What is the usual color of the WAN Port?

Usually Blue.

### What is the usual color of LAN Ports?

Usually Yellow.

(Note: Colors may vary depending on the manufacturer.)

# Troubleshooting Tips

## Power LED OFF

Check:

- Power Adapter

- Power Switch

- Power Socket

## LAN LED OFF

Check:

- Ethernet Cable

- LAN Port

- Network Interface Card (NIC)

## Internet LED OFF

Check:

- WAN Cable

- ISP Connection

## Ping Failed

Run

ipconfig

Then verify:

- Default Gateway

- Ethernet Cable

- Firewall Settings

# Day 3 Practical Commands

## View IP Address

ipconfig

## View Detailed Network Information

ipconfig /all

## Ping the Router

ping 192.168.1.1

## Ping Google Public DNS

ping 8.8.8.8

## Ping a Website

ping google.com

## Continuous Ping

ping 192.168.1.1 -t

### Purpose

Checks network stability continuously.

Press

Ctrl + C

to stop the test.

## Trace the Route

tracert google.com

Displays every router between your computer and Google's server.

## DNS Lookup

nslookup google.com

Displays:

- Website IP Address

- DNS Server

## View Routing Table

route print

Displays the routing table of the computer.

## View Network Statistics

netstat

Displays active network connections.

### Detailed Version

netstat -ano

Displays

- Process ID (PID)

- Port Number

- Connection Status

## View Shared Resources

net view

Displays shared computers and shared folders on the network.

## View Computer Name

hostname

Displays the computer's hostname.

## View System Information

systeminfo

Displays

- Operating System

- RAM

- BIOS

- Network Information

# Router Configuration Tips

Always connect the computer to a LAN Port, not the WAN Port.

Computer

│

LAN Port

If the computer does not receive an IP Address,

check whether DHCP is enabled.

If a Static IP Address is required, configure:

IP Address : 192.168.1.20

Subnet Mask : 255.255.255.0

Default Gateway : 192.168.1.1

DNS Server : 8.8.8.8

# Practical Checklist

✔ Router Power LED ON

✔ LAN LED ON

✔ WAN Connected

✔ IP Address Assigned

✔ Default Gateway Available

✔ Router Responds to Ping

✔ Router Login Page Opens

✔ Internet Connection Working

# Common Viva Questions

| Question | Answer |
| --- | --- |
| What is DHCP? | DHCP automatically assigns IP Address, Subnet Mask, Default Gateway, and DNS Server. |
| What is a Default Gateway? | It is the device (usually the router) that connects the local network to other networks or the Internet. |
| Which command displays the Default Gateway? | ipconfig |
| Which command displays the MAC Address? | getmac or ipconfig /all |
| Which command checks DNS information? | nslookup |
| Which command displays the Routing Table? | route print |
| Which command tests network connectivity? | ping |
| Which command traces the network path? | tracert |
| Which command clears the DNS cache? | ipconfig /flushdns |
| Which command releases the current IP Address? | ipconfig /release |
| Which command requests a new IP Address? | ipconfig /renew |
| Which command displays active network connections? | netstat -ano |

# Pro Tips (Real-World)

- Always connect the Router LAN Port to any normal port on the Switch. Most unmanaged switches do not require a dedicated uplink port.

- Run Command Prompt as Administrator because some networking commands require administrative privileges.

- If your computer receives an IP address like 169.254.x.x, it usually means the DHCP server did not assign an IP address. Check the Ethernet cable, switch port, router, and DHCP service.

- Avoid creating a network loop by connecting two cables between the same router and switch or between two unmanaged switches. This can cause a broadcast storm, making the network very slow or completely unavailable.

- Follow this troubleshooting sequence whenever a network problem occurs:

Power

↓

Check Ethernet Cable

↓

Check LED Indicators

↓

Run ipconfig

↓

Ping the Default Gateway

↓

Ping 8.8.8.8

↓

Ping google.com

- To restore a router to its factory settings, press and hold the Reset Button for approximately 10–15 seconds. (The exact time may vary depending on the router manufacturer.)

These practical commands, troubleshooting methods, and real-world tips are highly useful for CCNA, Networking Support Engineer, Desktop Support Engineer, Hardware & Networking Technician, and networking job interviews.