Day 5: Network Testing

Duration: 1 Hour

Practical Session

Topic: Connect Devices to a Network and Test Connectivity Using Ping and Traceroute

Learning Objectives

After completing this session, students will be able to:

• Connect devices to a network.
• Verify network connectivity.
• Use Ping to test communication between devices.
• Use Traceroute to identify the path taken by network traffic.
• Troubleshoot basic network connectivity issues.
• Prepare a report on setting up and testing a basic network.

Introduction

After a network has been installed, it must be tested to ensure all devices can communicate properly. Network testing helps identify connectivity problems and verifies that devices are configured correctly.

Network Testing Tools

- Ping

Ping is a command-line utility used to test whether a device can communicate with another device on a network.

Purpose

• Verify network connectivity.
• Check whether a device is reachable.
• Measure response time.

Command:

ping 192.168.1.1

Example Output:

Reply from 192.168.1.1

Packets Sent = 4

Packets Received = 4

Packets Lost = 0

Successful ping responses indicate that communication is working correctly.

- Traceroute (Tracert)

Traceroute displays the path that network packets take from the source device to the destination device.

Purpose

• Identify network routes.
• Locate connectivity problems.
• Analyze network paths.

Command:

tracert google.com

Example Output:

1 Router

2 ISP Gateway

3 Internet Backbone

4 Destination Server

Each step is called a Hop.

Practical Activity

Objective

Connect devices to a network and verify communication using network testing tools.

Requirements

• Router
• Switch (optional)
• Two Computers or Laptops
• Ethernet Cables
• Internet Connection (optional)

Network Setup

Computer 1

↓

Router or Switch

↓

Computer 2

Practical Procedure

Step 1: Connect Network Devices

• Connect both computers to the router or switch using Ethernet cables.
• Ensure all devices are powered on.

Step 2: Verify Physical Connectivity

Check LED Indicators:

• Power LED
• Link LED
• Activity LED

Ensure all connected ports show active status.

Step 3: Check IP Configuration

Open Command Prompt.

Type:

ipconfig

Record:

• IP Address
• Subnet Mask
• Default Gateway

Verify that both computers belong to the same network.

Example:

Computer 1: 192.168.1.10

Computer 2: 192.168.1.20

Subnet Mask: 255.255.255.0

Step 4: Test Router Connectivity

Open Command Prompt.

Type:

ping 192.168.1.1

Expected Result:

Successful replies from the router.

Step 5: Test Connectivity Between Computers

From Computer 1:

ping 192.168.1.20

Expected Result:

Replies received from Computer 2.

Step 6: Test Internet Connectivity

If internet access is available:

ping google.com

Expected Result:

Replies received from Google's server.

Step 7: Trace Network Route

Type:

tracert google.com

Observe:

• Number of hops
• Network route information

Step 8: Record Results

Document all observations and command outputs.

Common Connectivity Issues

Issue: No IP Address Assigned

Possible Cause:

• Faulty cable
• DHCP problem

Solution:

• Reconnect cable
• Restart router

Issue: Request Timed Out

Possible Cause:

• Device offline
• Firewall restriction

Solution:

• Verify device power
• Check network settings

Issue: Unable to Reach Internet

Possible Cause:

• Router configuration issue
• ISP connection problem

Solution:

• Verify router status
• Check WAN connection

Practical Task

Perform the following:

□ Connect devices to the network.

□ Verify IP addresses.

□ Ping the router.

□ Ping another computer.

□ Test internet connectivity.

□ Run tracert command.

□ Record observations.

Network Testing Report

Student Name: ____________________

Date: ____________________

Objective

To connect devices to a network and verify connectivity using network testing tools.

Equipment Used

• Router
• Computer/Laptop
• Ethernet Cable
• Network Switch (if available)

Procedure

- Connected devices to the router.

- Verified physical connections.

- Checked IP addresses using ipconfig.

- Tested connectivity using ping.

- Tested network route using tracert.

- Recorded observations.

Observations

Computer IP Address: ____________________

Default Gateway: ____________________

Router Ping Successful: Yes / No

Computer-to-Computer Ping Successful: Yes / No

Internet Ping Successful: Yes / No

Traceroute Completed: Yes / No

Result

The network was successfully configured and tested. Devices were able to communicate through the network and connectivity was verified using ping and traceroute commands.

Conclusion

This practical demonstrated how to connect devices to a network, verify connectivity, and troubleshoot basic communication issues using standard network testing tools.

Expected Outcome

Students should be able to connect devices to a network, verify network communication, use ping and tracert commands, and prepare a simple network testing report.