Day 2: Diagnosing Hardware Failures

Duration: 1 Hour

Theory Session (30 Minutes)

Topic: Common Hardware Issues and Their Symptoms

Learning Objectives

After completing this session, students will be able to:

• Identify common computer hardware failures.
• Recognize symptoms of faulty hardware components.
• Understand basic diagnostic techniques.
• Use a multimeter to test electrical components safely.

Introduction

Hardware failures can occur due to aging components, power fluctuations, overheating, physical damage, or manufacturing defects. Correctly identifying symptoms helps technicians diagnose and repair problems efficiently.

Common Hardware Failures and Their Symptoms

- Power Supply (SMPS) Failure

Symptoms

• Computer does not power on.
• No fan movement.
• No LED indicators.
• Random shutdowns or restarts.
• Burning smell from the system unit.

Possible Causes

• Faulty power supply.
• Loose power connections.
• Power surge damage.

- RAM Failure

Symptoms

• Continuous beep sounds during startup.
• Blue Screen of Death (BSOD).
• System freezes frequently.
• Computer fails to boot.

Possible Causes

• Defective RAM module.
• Dirty RAM contacts.
• Improper RAM installation.

- Hard Disk Drive (HDD) or SSD Failure

Symptoms

• Slow system performance.
• Operating system fails to load.
• Clicking noises from HDD.
• Missing files or data corruption.

Possible Causes

• Physical damage.
• Bad sectors.
• Aging storage device.

- CPU Failure

Symptoms

• No display on monitor.
• System does not complete startup.
• Unexpected shutdowns.
• Excessive overheating.

Possible Causes

• Faulty processor.
• Cooling system failure.
• Thermal paste issues.

- Motherboard Failure

Symptoms

• No power to system.
• No display output.
• USB ports not working.
• Random hardware detection problems.

Possible Causes

• Damaged circuits.
• Power surge.
• Faulty capacitors.

- Graphics Card (GPU) Failure

Symptoms

• No display.
• Distorted graphics.
• Screen flickering.
• System crashes during graphics-intensive tasks.

Possible Causes

• Overheating.
• Driver issues.
• Hardware damage.

- Cooling Fan Failure

Symptoms

• High CPU temperature.
• System overheating.
• Loud noise from fan.
• Automatic shutdowns.

Possible Causes

• Dust accumulation.
• Damaged fan motor.
• Loose power connection.

- CMOS Battery Failure

Symptoms

• Incorrect date and time.
• BIOS settings reset after shutdown.
• Startup warnings.

Possible Causes

• Weak or discharged CMOS battery.

Basic Hardware Diagnostic Process

Step 1

Identify the problem and gather information.

Step 2

Observe symptoms carefully.

Step 3

Inspect physical connections.

Step 4

Test suspected components.

Step 5

Replace or repair faulty hardware.

Introduction to Multimeter Testing

A multimeter is used to measure:

• Voltage
• Resistance
• Continuity

Multimeters help technicians identify electrical faults in computer hardware.

Practical Session (30 Minutes)

Topic: Testing Components Using a Multimeter

Objective

Students will learn how to use a multimeter to test basic computer-related electrical components safely.

Requirements

• Digital Multimeter
• Power Supply Unit (SMPS)
• CMOS Battery
• Power Cable
• Test Leads (Red and Black Probes)

Safety Precautions

• Never touch exposed electrical contacts while testing.
• Use the correct measurement mode.
• Handle probes carefully.
• Ensure hands are dry during testing.

Practical Activity 1: Identifying Multimeter Parts

Locate the following parts:

• Display Screen
• Function Selector Dial
• Red Probe
• Black Probe
• Voltage Port
• COM Port

Expected Result

Students should identify all major multimeter components.

Practical Activity 2: Testing a CMOS Battery

Objective

Measure the voltage of a CMOS battery.

Procedure

Step 1

Set the multimeter to DC Voltage.

Step 2

Place the black probe on the negative side.

Step 3

Place the red probe on the positive side.

Step 4

Read the voltage displayed.

Expected Result

A healthy CMOS battery should show approximately 3 volts.

Observation

Battery Voltage: ___________

Condition:

□ Good

□ Replace Battery

Practical Activity 3: Continuity Test on a Cable

Objective

Check whether a cable has an unbroken electrical path.

Procedure

Step 1

Set the multimeter to Continuity Mode.

Step 2

Touch both probes together to verify operation.

Step 3

Place probes on both ends of the cable.

Step 4

Listen for a beep or observe continuity indication.

Expected Result

The multimeter should indicate continuity if the cable is functioning correctly.

Observation

Continuity Present:

□ Yes

□ No

Practical Activity 4: Measuring SMPS Output Voltage (Demonstration)

Objective

Observe how a multimeter can be used to verify power supply output.

Procedure

Step 1

Instructor demonstrates voltage testing.

Step 2

Observe measurement points.

Step 3

Record displayed voltage values.

Common SMPS Output Values

• Yellow Wire = +12V
• Red Wire = +5V
• Orange Wire = +3.3V
• Black Wire = Ground

Expected Result

Voltage values should be close to their rated output.

Observation Sheet

Student Name: __________________

Date: __________________

CMOS Battery Voltage: ___________

Continuity Test Result: Pass / Fail

SMPS Voltage Observed: ___________

Any Abnormal Reading: ___________

Assessment Questions

- What is the purpose of a multimeter?

- Which multimeter mode is used to test continuity?

- What is the normal voltage of a CMOS battery?

- Name two symptoms of RAM failure.

- Which wire in an SMPS commonly provides 12V output?

Conclusion

Diagnosing hardware failures requires observing symptoms and using appropriate testing tools. A multimeter is one of the most important diagnostic instruments for checking voltage, continuity, and electrical health of computer components.