Day 4: Component-Specific Troubleshooting

Duration: 1 Hour

Practical Session

Topic: Identify and Fix Issues in Faulty Components (RAM, GPU, Storage Devices)

Learning Objectives

After completing this practical session, students will be able to:

• Identify common hardware component failures.
• Diagnose RAM, GPU, and storage device problems.
• Apply basic troubleshooting techniques.
• Verify whether a component is functioning correctly.
• Create and use a hardware troubleshooting checklist.

Introduction

Hardware components can fail due to physical damage, overheating, improper installation, power fluctuations, or normal wear and tear. A technician must systematically diagnose the problem before replacing any component.

Components Covered

• RAM (Memory)
• Graphics Card (GPU)
• Hard Disk Drive (HDD)
• Solid State Drive (SSD)

Practical Activity 1: RAM Troubleshooting

Objective

Identify and resolve memory-related issues.

Common Symptoms

• Computer does not boot.
• Continuous beep sounds during startup.
• Blue Screen of Death (BSOD).
• Frequent system freezes.
• Random restarts.

Procedure

Step 1

Power off the computer.

Step 2

Open the system cabinet.

Step 3

Locate the RAM module.

Step 4

Remove the RAM carefully.

Step 5

Inspect for dust or damage.

Step 6

Clean RAM contacts using a soft eraser or dry cloth.

Step 7

Reinstall the RAM securely.

Step 8

Power on the computer.

Expected Result

System boots successfully without memory-related errors.

Practical Activity 2: Graphics Card (GPU) Troubleshooting

Objective

Identify and resolve graphics card issues.

Common Symptoms

• No display on monitor.
• Screen flickering.
• Distorted graphics.
• System crashes during graphics-intensive tasks.

Procedure

Step 1

Power off the computer.

Step 2

Inspect the graphics card installation.

Step 3

Verify that the card is fully inserted into the PCIe slot.

Step 4

Check power connectors (if applicable).

Step 5

Clean dust from cooling fans and heatsinks.

Step 6

Reconnect the monitor cable.

Step 7

Power on the system.

Expected Result

Display appears normally without visual errors.

Practical Activity 3: Storage Device Troubleshooting

Objective

Identify storage device problems and verify connectivity.

Common Symptoms

• Operating system not found.
• Slow performance.
• Files missing or corrupted.
• Drive not detected.

Procedure

Step 1

Power off the computer.

Step 2

Locate the HDD or SSD.

Step 3

Check SATA and power connections.

Step 4

Reconnect cables securely.

Step 5

Power on the computer.

Step 6

Verify drive detection in BIOS.

Step 7

Open File Explorer and confirm drive accessibility.

Expected Result

Storage device is detected and accessible.

Practical Activity 4: Basic Visual Inspection

Objective

Identify visible hardware problems.

Check for:

• Loose cables.
• Dust accumulation.
• Damaged connectors.
• Burn marks.
• Broken cooling fans.
• Swollen capacitors.

Expected Result

Students identify any visible signs of hardware failure.

Task: Create a Checklist for Diagnosing Hardware Issues

Hardware Troubleshooting Checklist

General Inspection

□ Computer receives power.

□ Power cables are connected properly.

□ No burning smell is present.

□ Cooling fans are operating normally.

□ No loose cables inside the cabinet.

Power Supply Check

□ Power supply fan is spinning.

□ Power LED is illuminated.

□ Voltage outputs appear normal (if tested).

RAM Check

□ RAM is seated correctly.

□ RAM contacts are clean.

□ No physical damage is visible.

□ System boots without memory errors.

Graphics Card Check

□ GPU is properly installed.

□ Power connectors are attached.

□ Display cable is connected.

□ Display output is working correctly.

Storage Device Check

□ SATA cable connected securely.

□ Power cable connected securely.

□ Drive detected in BIOS.

□ Drive accessible in operating system.

Cooling System Check

□ CPU fan functioning.

□ No excessive dust.

□ Airflow unobstructed.

□ Temperature within normal range.

Peripheral Check

□ Keyboard functioning.

□ Mouse functioning.

□ Monitor functioning.

□ Printer or other devices detected.

Software Verification

□ Device Manager shows no hardware errors.

□ Operating system loads successfully.

□ Drivers installed correctly.

□ No critical error messages.

Observation Sheet

Student Name: __________________

Date: __________________

Component Tested: __________________

Issue Found: __________________

Diagnostic Method Used: __________________

Corrective Action Taken: __________________

Final Result: __________________

Assessment Questions

- What are two common symptoms of RAM failure?

- Why should hardware be powered off before inspection?

- What can cause a storage device to be undetected?

- Name two symptoms of GPU failure.

- Why is visual inspection important during troubleshooting?

Expected Outcome

Students should be able to identify hardware faults, apply corrective actions, verify component functionality, and use a structured troubleshooting checklist to diagnose computer hardware problems.

Conclusion

A systematic troubleshooting process helps technicians identify hardware problems quickly and accurately. By inspecting RAM, GPU, and storage devices, students gain practical experience in diagnosing and resolving common computer hardware issues.