Day 5: Case Study Practical

Duration: 1 Hour

Practical Session

Topic: Troubleshooting a Computer with Multiple Hardware Issues

Learning Objectives

After completing this practical session, students will be able to:

• Apply troubleshooting techniques in a real-world scenario.
• Identify multiple hardware faults.
• Follow a systematic diagnostic process.
• Record findings and recommend solutions.
• Improve problem-solving and analytical skills.

Introduction

In real-world environments, computers may experience multiple hardware problems at the same time. Technicians must diagnose issues methodically instead of assuming a single component is faulty.

This case study simulates a computer with several hardware-related problems. Students must inspect the system, identify faults, and recommend corrective actions.

Case Study Scenario

A customer reports the following issues:

• The computer occasionally fails to start.
• The system date and time reset after shutdown.
• The computer runs very slowly.
• The display sometimes does not appear.
• The system becomes unusually hot after a short period of use.

Your task is to diagnose the problems and suggest appropriate solutions.

Practical Procedure

Step 1: Initial Inspection

Check the following:

• Power cable connection
• Monitor connection
• Keyboard and mouse connection
• Cabinet condition
• Cooling fan operation

Record observations.

Step 2: Power-On Test

Start the computer and observe:

• Beep sounds
• POST messages
• Error messages
• Display output

Record observations.

Step 3: BIOS Verification

Enter BIOS Setup.

Check:

• System date and time
• Storage device detection
• CPU temperature (if available)

Record observations.

Step 4: Hardware Inspection

Inspect the following components:

RAM

• Properly installed
• Dust accumulation
• Physical damage

Storage Device

• SATA cable connection
• Power cable connection

Graphics Card

• Proper seating in PCIe slot
• Power connector attached

Cooling System

• CPU fan operation
• Dust in heatsink
• Airflow blockage

CMOS Battery

• Check battery condition
• Verify BIOS settings retention

Step 5: Software-Based Diagnostics

Open Task Manager.

Check:

• CPU usage
• Memory usage
• Disk usage

Open Device Manager.

Check:

• Hardware errors
• Missing drivers

Step 6: Identify Problems

Based on observations, determine the likely causes.

Possible Findings

Issue 1

Symptom:

System date and time reset after shutdown.

Diagnosis:

Weak CMOS battery.

Solution:

Replace CMOS battery.

Issue 2

Symptom:

Computer runs slowly.

Diagnosis:

Low available memory or storage issues.

Solution:

Check RAM and storage health.

Issue 3

Symptom:

No display appears occasionally.

Diagnosis:

Loose graphics card or display cable.

Solution:

Reseat graphics card and reconnect cables.

Issue 4

Symptom:

System overheating.

Diagnosis:

Dust accumulation or faulty cooling fan.

Solution:

Clean cooling system and verify fan operation.

Issue 5

Symptom:

Startup problems.

Diagnosis:

Loose RAM module or power issue.

Solution:

Reseat RAM and inspect power connections.

Case Study Analysis Sheet

Student Name: __________________

Date: __________________

Observed Symptoms

Problems Identified

Recommended Solutions

Final Result

Task: Hardware Troubleshooting Report

Prepare a short report including:

- Problem Description

Describe the issues reported by the customer.

- Inspection Performed

List all checks completed during troubleshooting.

- Faults Identified

Mention the faulty components or issues found.

- Corrective Actions

Explain the steps taken to resolve each problem.

- Result

State whether the system is functioning properly after troubleshooting.

Assessment Questions

- Why should troubleshooting follow a step-by-step process?

- Which component can cause date and time reset issues?

- What are common causes of overheating?

- What symptoms may indicate a RAM problem?

- Why is physical inspection important during troubleshooting?

Expected Outcome

Students should be able to analyze multiple hardware problems, identify root causes, recommend solutions, and document the troubleshooting process professionally.

Conclusion

Successful troubleshooting requires observation, logical thinking, and systematic testing. By working through a real-world case study, students gain practical experience in diagnosing and resolving multiple hardware issues simultaneously.