# A+ Hardware & System Troubleshooting

## 1. What is Troubleshooting?

Troubleshooting means:

Find the problem → Find the reason → Fix the problem → Test again

Real-time example:

A computer is not starting.

Don't directly say:

"Motherboard is damaged."

First check:

Power Cable

↓

Power Socket

↓

SMPS

↓

Power Button

↓

Motherboard

↓

RAM

↓

CPU

This is troubleshooting.

# 2. Troubleshooting Rule

Teach students this simple rule:

### First check the easiest things.

Example:

Monitor is not showing anything.

Don't immediately remove the CPU.

Check:

- Monitor power ON?

- HDMI/VGA/DP cable connected?

- Correct input selected?

- Brightness?

- Cable working?

- Try another monitor.

Only then check RAM, GPU, motherboard, etc.

# 3. PC Does Not Turn ON

### Problem

Student presses power button but nothing happens.

### Possible Causes

- Power cable disconnected

- Power socket OFF

- UPS problem

- SMPS switch OFF

- SMPS failure

- Cabinet power button problem

- Motherboard power connection problem

### Troubleshooting Steps

Check socket

↓

Check power cable

↓

Check UPS

↓

Check SMPS switch

↓

Check motherboard 24-pin connection

↓

Check CPU power connection

↓

Check Power SW connection

↓

Test SMPS

### Real-time Question

Ask students:

"If there is absolutely no fan movement and no LED, will you first check RAM?"

Expected answer:

No. First check power.

# 4. PC Turns ON but No Display

This is a very common A+ troubleshooting problem.

### Situation

Fans are running:

Power ON

Fans ON

Keyboard light ON

But Monitor = No Signal

Possible causes:

- RAM not properly inserted

- RAM faulty

- Display cable problem

- Monitor problem

- GPU problem

- Wrong display port

- CPU/motherboard problem

### Troubleshooting Order

Monitor ON?

↓

Cable connected?

↓

Correct input?

↓

Try another cable

↓

Try another monitor

↓

Reseat RAM

↓

Test RAM one by one

↓

Check GPU

### Practical Activity

If training lab has spare RAM:

- Shut down PC.

- Remove power.

- Remove RAM.

- Clean/check contacts carefully as appropriate.

- Reinsert RAM properly.

- Start PC.

- Check display.

Explain:

Loose RAM is a common reason for a PC that powers on but doesn't display.

# 5. PC Beeps During Startup

Suppose:

PC ON

↓

Screen blank

↓

Beep sound

Explain that POST checks hardware during startup.

POST can detect hardware problems such as:

- RAM

- Keyboard

- Video

- CPU

- Other motherboard-related hardware

Important:

Beep patterns depend on the motherboard/BIOS/UEFI, so students should check the system's documentation rather than memorize one universal beep code.

# 6. PC Restarts Again and Again

### Real-time situation

Student says:

"Sir, PC starts and after 5 minutes it automatically restarts."

Possible causes:

- CPU overheating

- Dust

- CPU fan problem

- RAM problem

- SMPS problem

- Driver/software problem

- Windows issue

- Hardware fault

### Troubleshooting

First check:

CPU temperature

CPU fan

Dust

Airflow

RAM

Power supply

Windows/Event logs

### Real-life Example

If CPU fan is not spinning:

CPU gets hot

↓

System becomes unstable

↓

PC may shut down/restart

# 7. Computer Becomes Very Hot

Ask students:

"Why does a computer become hot?"

Possible reasons:

- Dust inside cabinet

- CPU fan not working

- Blocked air vents

- Thermal paste issue

- Poor airflow

- High CPU workload

### Practical Observation

Without opening anything, students can observe:

- Cabinet ventilation

- CPU fan operation

- Dust

- Air vents

- Noise from fans

Teach them:

Heat is an important hardware troubleshooting clue.

# 8. PC Makes Strange Noise

### Problem

Computer makes:

Clicking

Grinding

Loud fan noise

Beeping

### Possible Causes

Loud fan noise

→ Dust / fan bearing / high temperature

Clicking HDD

→ Possible mechanical disk problem

Beep during startup

→ Possible POST hardware issue

Grinding

→ Fan or mechanical drive issue

### Real-time example

A student says:

"My PC sounds like a small machine."

First check:

- Which component is making the noise?

- CPU fan?

- Cabinet fan?

- GPU fan?

- HDD?

Don't randomly replace components.

# 9. Hard Disk / SSD Problem

### Situation

Windows becomes very slow.

Possible reasons:

- Low free space

- HDD aging/failure

- Too many startup applications

- High disk usage

- Windows issue

### Windows Practical

Open:

Task Manager

Check:

Disk: 100%

Ask students:

"Does 100% disk usage automatically mean the hard disk is damaged?"

Answer:

No.

It means disk activity is currently very high. We need to investigate the cause.

# 10. Low Disk Space

### Situation

Windows shows:

"Low disk space"

Check:

This PC

↓

C:

↓

Free Space

Possible causes:

- Large downloads

- Temporary files

- Old installers

- Recycle Bin

- Large videos

- Windows update files

### Troubleshooting

Use:

Settings → System → Storage

or:

Win + R

Then:

cleanmgr

Students can see Windows Disk Cleanup.

# 11. RAM Problem

### Symptoms

- PC becomes slow

- Applications crash

- Windows freezes

- Blue screen

- PC doesn't boot

- No display in some cases

### Real-time Example

Computer has:

4 GB RAM

Student opens:

Chrome

Eclipse

Android Studio

Photoshop

System becomes very slow.

Ask:

"Is the RAM necessarily faulty?"

No.

It may simply be insufficient for the workload.

This is an important difference:

Faulty RAM ≠ Low RAM

# 12. Keyboard Not Working

### Situation

Keyboard does not work.

Check:

USB connection

↓

Try another USB port

↓

Try another keyboard

↓

Check Device Manager

↓

Check driver

For wireless keyboard:

- Battery

- USB receiver

- Pairing

### Real-time Example

If keyboard works on another PC:

The keyboard itself may not be the problem.

This teaches students to isolate the faulty component.

# 13. Mouse Not Working

Check:

- USB connection

- Different USB port

- Mouse sensor

- Try another mouse

- Device Manager

- Driver

### Practical

Students can remove the mouse and connect it to another PC.

Ask:

"What did we just do?"

Answer:

We isolated the problem.

# 14. USB Port Not Working

### Problem

Pendrive works on one USB port but not another.

Example:

Front USB → Not working

Rear USB  → Working

Possible causes:

- Front USB cable disconnected internally

- Damaged USB port

- Driver problem

- Power issue

### Important Troubleshooting Rule

If the same pendrive works elsewhere:

The pendrive may be okay.

Now test the USB port.

# 15. Printer Hardware Troubleshooting

Pure A+ hardware example.

### Problem

Printer doesn't print.

Check:

Power

↓

Paper

↓

Ink/Toner

↓

USB/LAN/Wi-Fi connection

↓

Paper jam

↓

Printer status

### Real-time Example

Student says:

"Printer is not working."

You ask:

"Is the printer ON?"

Student:

"No."

This sounds funny, but this is exactly why troubleshooting starts with the basics.

# 16. Monitor Problem

### Situation

Monitor shows:

No Signal

Troubleshooting:

Monitor Power

↓

Display Cable

↓

Correct Input Source

↓

Cable Replacement

↓

Different Monitor

↓

RAM

↓

GPU

↓

Motherboard

### Important

If PC is running but monitor says:

No Signal

It does not automatically mean monitor is faulty.

The problem can be:

- Cable

- GPU

- RAM

- Motherboard

- Wrong input

- PC not actually completing POST

# 17. Windows Does Not Boot

### Situation

Power ON → Manufacturer logo → Windows doesn't start.

Possible causes:

- Boot configuration problem

- Corrupted Windows files

- Storage problem

- Update problem

- Boot device issue

### Troubleshooting Flow

Power ON

↓

POST successful?

↓

BIOS/UEFI detects storage?

↓

Windows starts?

↓

Error message?

↓

Startup Repair / Recovery options

Teach students to read the exact error message.

# 18. Windows Blue Screen

### Problem

Windows shows a blue-screen error.

Don't immediately reinstall Windows.

First:

- Note the error/message.

- Think about recent hardware/software changes.

- Check drivers.

- Check RAM/storage.

- Check Windows logs.

- Search the exact error code when appropriate.

### Real-time example

Student installed a new driver yesterday.

Today:

Blue screen started.

That timing is an important troubleshooting clue.

# 19. Device Manager Troubleshooting

This is a must-have A+ practical.

Open:

Win + X

↓

Device Manager

Look for devices with warning indicators.

Check:

- Display adapters

- Network adapters

- Disk drives

- Keyboards

- Mice

- USB controllers

### Real-time Example

If a device has a driver problem:

Hardware

↓

Device Manager

↓

Device status

↓

Driver

Students should learn to check Device Manager before assuming hardware is physically damaged.

# 20. Windows System Information

Run:

Win + R

Type:

msinfo32

Students can see information about:

- System model

- Processor

- RAM

- BIOS

- Windows version

- Hardware resources

Another useful command:

systeminfo

This gives system information from CMD.

# 21. One Very Important A+ Concept

Teach this sentence:

Symptoms do not always tell you the exact cause.

Example:

### Symptom:

Computer is slow.

Possible causes:

Low RAM

High CPU usage

High disk usage

Low disk space

Malware

Too many startup programs

Overheating

Old hardware

So:

Symptom ≠ Cause

Troubleshooting means finding the actual cause.

# 22. Best Classroom Practical: "Troubleshooting Doctor"

You can make today's lecture very interactive.

Tell students:

"You are the computer technician. I am the customer."

### Case 1

Customer:

"My computer is not turning ON."

Students must ask questions.

You answer:

"No fan. No LED."

Students should check:

Power first.

### Case 2

Customer:

"Computer turns ON but display is not coming."

You answer:

"Fans are running and monitor says No Signal."

Students should investigate:

Monitor → cable → input → RAM → GPU

### Case 3

Customer:

"Computer is very slow."

You answer:

"Task Manager shows Memory 95%."

Students identify:

High memory usage.

### Case 4

Customer:

"Computer suddenly shuts down."

You answer:

"CPU temperature is very high."

Students identify:

Overheating as a possible cause.

### Case 5

Customer:

"Pendrive works on another computer but not mine."

Students should identify:

Computer-side USB port/driver issue is more likely than the pendrive itself, but it still needs testing.

# Final Board Summary

Write this on the board:

A+ TROUBLESHOOTING

1. Identify the problem

2. Ask questions

3. Check simple things first

4. Check power

5. Check physical connections

6. Check hardware

7. Check Windows/System

8. Read error messages

9. Isolate the faulty component

10. Fix the problem

11. Test again

12. Document the solution

And one golden line:

Don't guess the problem. Test the problem.