Day 1: Tools for Troubleshooting

Duration: 1 Hour

Theory Session

Topic: Overview of Troubleshooting Tools (Multimeter, POST Card, Software Diagnostics)

Learning Objectives

After completing this session, students will be able to:

• Understand the importance of troubleshooting tools.
• Identify commonly used hardware troubleshooting tools.
• Explain the purpose of a multimeter.
• Understand how a POST card is used for computer diagnostics.
• Recognize software-based diagnostic tools.
• Select the appropriate tool for troubleshooting different computer problems.

Introduction

Troubleshooting is the process of identifying, diagnosing, and resolving problems in computer hardware and software. Technicians use various tools to detect faults quickly and accurately.

Common Troubleshooting Tools

Hardware Troubleshooting Tools

• Multimeter
• POST Card
• Screwdriver Set
• Power Supply Tester
• Anti-Static Wrist Strap

Software Troubleshooting Tools

• Task Manager
• Device Manager
• Event Viewer
• System Information
• Windows Memory Diagnostic
• Command Prompt Utilities

- Multimeter

Definition

A multimeter is an electronic measuring instrument used to test electrical values such as voltage, resistance, and continuity.

Purpose

• Check power supply output.
• Test cables and connectors.
• Verify electrical continuity.
• Identify power-related problems.

Functions of a Multimeter

Voltage Measurement

Used to measure electrical voltage.

Example:

Testing power supply outputs.

Resistance Measurement

Used to measure resistance in electronic components.

Continuity Testing

Used to determine whether a circuit path is complete.

Common Multimeter Parts

• Display Screen
• Function Selector Dial
• Red Probe
• Black Probe
• Measurement Ports

Applications in Computer Troubleshooting

• Testing SMPS voltage output.
• Checking power cables.
• Identifying faulty connections.
• Verifying battery voltage.

Advantages

• Easy to use.
• Accurate measurements.
• Essential for hardware technicians.

- POST Card

Definition

POST stands for Power-On Self-Test.

A POST card is a diagnostic tool used to identify motherboard and hardware startup problems.

Purpose

• Detect hardware failures during system startup.
• Display POST error codes.
• Assist in diagnosing motherboard issues.

How a POST Card Works

When a computer starts:

- The BIOS performs a Power-On Self-Test.

- The POST card receives diagnostic codes.

- The codes are displayed on the card.

- Technicians use the codes to identify hardware faults.

Common Problems Identified by POST Cards

• RAM failure
• CPU failure
• Motherboard issues
• BIOS errors
• Expansion card problems

Advantages

• Fast diagnosis of startup failures.
• Useful when the system does not display anything on the monitor.
• Saves troubleshooting time.

- Software Diagnostic Tools

Software diagnostic tools help technicians identify system problems without opening the computer.

A. Task Manager

Purpose

• Monitor running applications.
• View CPU, RAM, Disk, and Network usage.
• End unresponsive programs.

How to Open

Press:

Ctrl + Shift + Esc

B. Device Manager

Purpose

• View installed hardware devices.
• Detect driver problems.
• Update or uninstall drivers.

How to Open

Press:

Windows + X

Select:

Device Manager

C. Event Viewer

Purpose

• Displays system logs.
• Records warnings and errors.
• Helps identify software and hardware issues.

How to Open

Press:

Windows + R

Type:

eventvwr.msc

D. System Information

Purpose

• Displays hardware and operating system information.

How to Open

Press:

Windows + R

Type:

msinfo32

E. Windows Memory Diagnostic

Purpose

• Tests RAM for memory errors.

How to Open

Press:

Windows + R

Type:

mdsched.exe

Common Troubleshooting Scenarios

Scenario 1

Problem:

Computer does not power on.

Tool Used:

Multimeter

Reason:

Check power supply voltage and connections.

Scenario 2

Problem:

Computer powers on but no display appears.

Tool Used:

POST Card

Reason:

Identify startup hardware errors.

Scenario 3

Problem:

System becomes slow.

Tool Used:

Task Manager

Reason:

Check CPU and memory usage.

Scenario 4

Problem:

Printer or device not detected.

Tool Used:

Device Manager

Reason:

Verify drivers and hardware status.

Scenario 5

Problem:

Unexpected system crashes.

Tool Used:

Event Viewer

Reason:

Review system error logs.

Comparison of Troubleshooting Tools

Multimeter

Purpose:
Electrical testing and measurements.

POST Card

Purpose:
Diagnosing motherboard and startup issues.

Task Manager

Purpose:
Monitoring system performance.

Device Manager

Purpose:
Managing hardware devices and drivers.

Event Viewer

Purpose:
Analyzing system errors and logs.

System Information

Purpose:
Viewing detailed system specifications.

Summary

• Troubleshooting tools help identify and resolve computer problems.
• A multimeter is used for electrical measurements.
• A POST card helps diagnose hardware startup issues.
• Software diagnostic tools assist in detecting system and driver problems.
• Choosing the correct tool improves troubleshooting efficiency.

Conclusion

Troubleshooting is an essential skill for computer technicians. Understanding how to use multimeters, POST cards, and software diagnostic tools enables technicians to diagnose hardware and software issues accurately and efficiently.