# Day 2: DHCP Configuration

Duration: 1 Hour
Theory: 30 Minutes
Practical: 30 Minutes

## 1. Learning Objectives

By the end of this session, students will be able to:

- Understand DHCP and its purpose in networking.

- Explain the DHCP working process (Discover, Offer, Request, Acknowledge).

- Understand DHCP server, client, and IP address assignment.

- Configure a DHCP server in Cisco Packet Tracer.

- Verify automatic IP address assignment.

- Create a diagram showing the DHCP process flow.

# Part A: Theory (30 Minutes)

## 2. What is DHCP?

DHCP stands for Dynamic Host Configuration Protocol.

DHCP is a network protocol that automatically assigns IP addresses and other network configuration details to devices connected to a network.

When a computer, mobile phone, or laptop connects to a network, it needs an IP address to communicate with other devices.

Instead of manually configuring an IP address on every device, DHCP automatically provides the required network configuration.

Real-time example:

When you connect your mobile phone to Wi-Fi:

- Your mobile phone acts as a DHCP client.

- Your Wi-Fi router generally acts as the DHCP server.

- The router assigns an available IP address to your mobile phone.

- Your mobile phone can communicate with other devices on the network.

## 3. Why Do We Need DHCP?

Without DHCP, a network administrator may need to manually configure the IP address of every device.

For example, in a computer lab with 50 computers, manually assigning IP addresses to all computers takes time.

DHCP makes this process automatic.

Benefits of DHCP:

- Automatically assigns IP addresses.

- Reduces manual configuration.

- Helps prevent duplicate IP address assignments.

- Provides subnet mask and default gateway information.

- Can provide DNS server information.

- Makes network management easier.

## 4. DHCP Components

DHCP communication mainly involves two components.

### DHCP Client

A DHCP client is a device that requests network configuration from a DHCP server.

Examples:

- Laptop

- Desktop computer

- Mobile phone

- Network printer

### DHCP Server

A DHCP server provides IP addresses and network configuration to DHCP clients.

Examples:

- Wi-Fi router

- Windows Server

- Linux DHCP server

- Cisco router configured for DHCP

### DHCP Address Pool

An address pool is a range of IP addresses that the DHCP server can assign to clients.

Example:

Network:       192.168.10.0/24

DHCP Pool:     192.168.10.100 - 192.168.10.200

Default Gateway: 192.168.10.1

DNS Server:    8.8.8.8

In this example, the DHCP server can assign IP addresses from the configured pool to network clients.

## 5. DHCP Working Process (DORA)

DHCP uses four main steps to assign an IP address.

DORA stands for:

- Discover

- Offer

- Request

- Acknowledge

DHCP Client

Computer / Laptop

DHCP Server

Router / Server

D

Discover

Client broadcasts a request to find a DHCP server.

O

Offer

Server offers an available IP address.

R

Request

Client requests the offered IP address.

A

Acknowledge

Server confirms the IP assignment.

### Step 1: Discover

When a client connects to a network, it does not initially know its IP address.

The client broadcasts a DHCP Discover message to search for a DHCP server.

### Step 2: Offer

The DHCP server receives the Discover message.

It checks its available IP address pool and offers an available IP address to the client.

Example:

Offered IP: 192.168.10.101

Subnet Mask: 255.255.255.0

Gateway:     192.168.10.1

### Step 3: Request

The client receives the DHCP Offer.

The client sends a DHCP Request message to request the offered IP address.

This message also informs DHCP servers which offer the client has selected.

### Step 4: Acknowledge

The DHCP server sends a DHCP Acknowledge (ACK) message.

The server confirms the IP address assignment.

The client can now use the assigned IP address and other network configuration details.

## 6. DHCP Ports

| Component | Protocol | Port |
| --- | --- | --- |
| DHCP Server | UDP | 67 |
| DHCP Client | UDP | 68 |

DHCP uses UDP because the client needs to communicate with the server before it has a fully configured IP address.

## 7. DHCP Process Flow Diagram

Example:

A computer joins a network and receives the IP address 192.168.10.101 from the DHCP server.

The computer can now communicate with other devices on the network.

# Part B: Practical (30 Minutes)

## Practical: Configure a DHCP Server in Cisco Packet Tracer

Objective: Configure a Cisco router as a DHCP server and automatically assign IP addresses to computers.

### Requirements

- Cisco Packet Tracer

- 1 Router

- 1 Switch

- 4 PCs

- Copper straight-through cables

## Step 1: Create the Network Topology

Open Cisco Packet Tracer.

Add the following devices:

- Router: 1941

- Switch: 2960

- PC0

- PC1

- PC2

- PC3

Connect the devices using copper straight-through cables.

### Connection Details

| Device | Port | Connected To |
| --- | --- | --- |
| Router0 | GigabitEthernet0/0 | Switch0 Fa0/1 |
| PC0 | FastEthernet0 | Switch0 Fa0/2 |
| PC1 | FastEthernet0 | Switch0 Fa0/3 |
| PC2 | FastEthernet0 | Switch0 Fa0/4 |
| PC3 | FastEthernet0 | Switch0 Fa0/5 |

## Step 2: Configure the Router Interface

Click Router0 → CLI.

Enter the following commands:

enable

configure terminal

interface gigabitEthernet0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

### Explanation

- enable enters privileged EXEC mode.

- configure terminal enters configuration mode.

- interface gigabitEthernet0/0 selects the router interface.

- ip address assigns the router's IP address.

- no shutdown enables the interface.

The router interface will act as the default gateway for the network.

## Step 3: Configure the DHCP Address Pool

Enter the following commands on Router0:

configure terminal

ip dhcp excluded-address 192.168.10.1 192.168.10.20

ip dhcp pool STUDENT_LAB

network 192.168.10.0 255.255.255.0

default-router 192.168.10.1

dns-server 8.8.8.8

exit

### Explanation

Excluded addresses

ip dhcp excluded-address 192.168.10.1 192.168.10.20

These addresses are reserved and will not be assigned from the DHCP pool.

DHCP pool

ip dhcp pool STUDENT_LAB

Creates a DHCP pool named STUDENT_LAB.

Network

network 192.168.10.0 255.255.255.0

Defines the network from which the router can assign IP addresses.

Default router

default-router 192.168.10.1

Provides the default gateway address to DHCP clients.

DNS server

dns-server 8.8.8.8

Provides a DNS server address to DHCP clients.

## Step 4: Configure PCs for DHCP

Perform these steps on every PC.

- Click PC0.

- Open Desktop.

- Select IP Configuration.

- Select DHCP.

Repeat for PC1, PC2, and PC3.

The PCs should automatically receive IP addresses.

Example:

| Device | IP Address |
| --- | --- |
| Router0 | 192.168.10.1 |
| PC0 | 192.168.10.21 |
| PC1 | 192.168.10.22 |
| PC2 | 192.168.10.23 |
| PC3 | 192.168.10.24 |

The exact addresses may differ depending on the DHCP lease state and configuration.

## Step 5: Verify DHCP Configuration

On Router0, execute:

show ip dhcp binding

This command displays the IP addresses assigned to DHCP clients.

Check the DHCP pool:

show ip dhcp pool

Check interface status:

show ip interface brief

The router interface should show an operational status.

## Step 6: Test Communication

On PC0:

- Click Desktop.

- Open Command Prompt.

- Execute:

ipconfig

Check the assigned IP address, subnet mask, and default gateway.

Now test the router connection:

ping 192.168.10.1

Repeat the ping test from other PCs.

Expected result: The PCs receive IP addresses automatically and can ping the router's gateway.

# Part C: Student Assignment

## Task: Create a Diagram Showing the DHCP Process Flow

Students must create a diagram explaining how a DHCP client receives an IP address.

### Diagram Requirements

Include the following:

- DHCP Client

- DHCP Server

- DHCP Discover

- DHCP Offer

- DHCP Request

- DHCP Acknowledge

- Assigned IP address

### Example Diagram

### Assignment Questions

- What is DHCP?

- Why do we use DHCP in a network?

- Explain the four steps of the DHCP process.

- What is the difference between a DHCP client and a DHCP server?

- What is the purpose of a DHCP address pool?

- Which UDP ports are used by DHCP?

- What is the role of the default gateway in DHCP?

- What happens if a DHCP server is not available?

### Submission Requirements

- Submit the DHCP process flow diagram.

- Submit a screenshot of the Packet Tracer topology.

- Submit screenshots showing IP addresses assigned to the PCs.

- Submit the router DHCP configuration.

- Submit the successful ping test.

## End-of-Day Quick Revision

| Topic | Quick Answer |
| --- | --- |
| DHCP full form | Dynamic Host Configuration Protocol |
| DHCP purpose | Automatically assigns network configuration |
| DHCP process | Discover → Offer → Request → Acknowledge |
| DHCP server port | UDP 67 |
| DHCP client port | UDP 68 |
| DHCP client example | Laptop |
| DHCP server example | Router |
| DHCP verification | show ip dhcp binding |
| IP verification | ipconfig |
| Connectivity test | ping |