# Day 3: Switch and Router Configuration Basics

## 1. Introduction

Today we will learn two important networking concepts:

- VLAN

- Static Routing

We will use Cisco Packet Tracer for practical work.

Basic network:

PCs → Switch → Router → Router → Other Network

# Part 1: VLAN

## 2. What is a VLAN?

VLAN means:

Virtual Local Area Network

A VLAN allows us to logically divide one physical switch into multiple separate networks.

### Simple Example

Suppose a college has one switch:

Switch

/       |       \

PC1      PC2      PC3

PC1 and PC2 are for the Computer Lab.

PC3 is for the Admin Department.

Instead of buying separate physical switches, we can create VLANs:

VLAN 10 → Computer Lab

VLAN 20 → Admin

Now the switch can logically separate these devices.

# 3. Why Do We Use VLANs?

VLANs are useful for:

- Network separation

- Security

- Department-wise organization

- Reducing broadcast domains

- Better network management

### Real-Life College Example

A college may have:

VLAN 10 → Students

VLAN 20 → Faculty

VLAN 30 → Admin

VLAN 40 → Server

Even if all devices are connected to the same physical switch, VLANs logically separate them.

# 4. VLAN Example

Suppose we have:

Switch

/     |     \

PC1    PC2    PC3

Configure:

PC1 → VLAN 10

PC2 → VLAN 10

PC3 → VLAN 20

PC1 and PC2 belong to the same VLAN.

PC3 belongs to a different VLAN.

# 5. Important VLAN Terms

## VLAN ID

Every VLAN has a number.

Examples:

VLAN 10

VLAN 20

VLAN 30

## Access Port

An access port normally connects an end device such as:

- PC

- Printer

- Server

Example:

PC1 → Switch Port Fa0/1

Fa0/1 can be configured as an access port for VLAN 10.

## Trunk Port

A trunk port carries traffic for multiple VLANs between network devices.

Example:

Switch 1 ===== Trunk ===== Switch 2

or:

Switch ===== Trunk ===== Router

A trunk is commonly used when multiple VLANs need to travel across one physical link.

# 6. VLAN Configuration Example

We will create:

VLAN 10 → LAB

VLAN 20 → ADMIN

Network:

Switch

/    |    \

PC1   PC2   PC3

|      |     |

VLAN10  VLAN10 VLAN20

# 7. Cisco Packet Tracer Practical – Create VLANs

## Step 1: Add Devices

Open Cisco Packet Tracer.

Add:

- 1 Switch

- 3 PCs

Use a switch such as:

2960

Connect:

PC1 → Switch Fa0/1

PC2 → Switch Fa0/2

PC3 → Switch Fa0/3

Use Copper Straight-Through cables.

# 8. Step 2: Configure IP Addresses

### PC1

IP Address: 192.168.10.10

Subnet Mask: 255.255.255.0

### PC2

IP Address: 192.168.10.20

Subnet Mask: 255.255.255.0

### PC3

IP Address: 192.168.20.10

Subnet Mask: 255.255.255.0

For this basic VLAN practical, a default gateway is not required yet.

# 9. Step 3: Open Switch CLI

Click the switch.

Go to:

CLI

Enter:

enable

Then:

configure terminal

# 10. Step 4: Create VLAN 10

vlan 10

name LAB

exit

Now VLAN 10 is created.

# 11. Step 5: Create VLAN 20

vlan 20

name ADMIN

exit

Now we have:

VLAN 10 → LAB

VLAN 20 → ADMIN

# 12. Step 6: Assign PC1 to VLAN 10

PC1 is connected to FastEthernet0/1.

Run:

interface fastEthernet 0/1

switchport mode access

switchport access vlan 10

exit

# 13. Step 7: Assign PC2 to VLAN 10

interface fastEthernet 0/2

switchport mode access

switchport access vlan 10

exit

# 14. Step 8: Assign PC3 to VLAN 20

interface fastEthernet 0/3

switchport mode access

switchport access vlan 20

exit

# 15. Step 9: Check VLANs

Run:

show vlan brief

You should see something similar to:

VLAN Name                             Status    Ports

---- -------------------------------- --------- -------------------------------

1    default                          active    Fa0/4, Fa0/5...

10   LAB                              active    Fa0/1, Fa0/2

20   ADMIN                            active    Fa0/3

This confirms that the ports have been assigned to the correct VLANs.

# 16. Test VLAN Communication

From PC1:

ping 192.168.10.20

PC1 and PC2 are in VLAN 10.

They should be able to communicate if their IP configuration and switch connections are correct.

Now from PC1:

ping 192.168.20.10

PC3 is in VLAN 20.

The ping should not work with this basic Layer 2 configuration, because VLAN 10 and VLAN 20 are separate networks and there is no Layer 3 routing configured between them.

This is an important concept:

VLAN 10

PC1 ↔ PC2

X

VLAN 20

PC3

To communicate between VLANs, we need inter-VLAN routing using a router or Layer 3 switch.

# 17. Useful VLAN Commands

### Display VLANs

show vlan brief

### Display interfaces

show interfaces status

### Display switch configuration

show running-config

### Check a specific interface

show running-config interface fastEthernet 0/1

# Part 2: Static Routing

# 18. What is Routing?

Routing means deciding where an IP packet should go to reach another network.

Example:

PC1

|

Router 1

|

Router 2

|

PC2

PC1 and PC2 are on different networks.

The routers need routes to know where to forward packets.

# 19. What is a Static Route?

A static route is a route that an administrator manually configures on a router.

Example:

Router 1

|

| 10.0.0.0/30

|

Router 2

Router 1 can be manually told:

"To reach Network 192.168.20.0/24, send packets to Router 2."

# 20. Static Routing Example

We will create this network:

PC1 -------- R1 -------- R2 -------- PC2

|           |

Network A    Network B

More specifically:

PC1

|

192.168.10.0/24

|

R1

|

10.0.0.0/30

|

R2

|

192.168.20.0/24

|

PC2

# 21. IP Address Plan

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| PC1 | NIC | 192.168.10.10 | 255.255.255.0 |
| R1 | G0/0 | 192.168.10.1 | 255.255.255.0 |
| R1 | G0/1 | 10.0.0.1 | 255.255.255.252 |
| R2 | G0/0 | 10.0.0.2 | 255.255.255.252 |
| R2 | G0/1 | 192.168.20.1 | 255.255.255.0 |
| PC2 | NIC | 192.168.20.10 | 255.255.255.0 |

The exact interface names may vary depending on the router model in Packet Tracer.

# 22. Configure PC1

Set:

IP Address:

192.168.10.10

Subnet Mask:

255.255.255.0

Default Gateway:

192.168.10.1

# 23. Configure PC2

Set:

IP Address:

192.168.20.10

Subnet Mask:

255.255.255.0

Default Gateway:

192.168.20.1

# 24. Configure Router 1

Open R1 CLI.

enable

configure terminal

Configure the LAN interface.

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Configure the connection toward R2:

interface gigabitEthernet 0/1

ip address 10.0.0.1 255.255.255.252

no shutdown

exit

# 25. Configure Router 2

Open R2 CLI.

enable

configure terminal

Configure the interface toward R1:

interface gigabitEthernet 0/0

ip address 10.0.0.2 255.255.255.252

no shutdown

exit

Configure the LAN interface:

interface gigabitEthernet 0/1

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

# 26. Check Interfaces

On R1:

show ip interface brief

On R2:

show ip interface brief

Look for:

Status: up

Protocol: up

If an interface is administratively down, check whether:

no shutdown

was configured.

# 27. Check Connected Networks

On R1:

show ip route

You should see its directly connected networks, such as:

192.168.10.0/24

10.0.0.0/30

On R2:

show ip route

You should see:

10.0.0.0/30

192.168.20.0/24

But R1 does not automatically know how to reach 192.168.20.0/24 just because R2 knows about it.

This is where static routing is required.

# 28. Configure Static Route on R1

On R1:

ip route 192.168.20.0 255.255.255.0 10.0.0.2

Meaning:

To reach:

192.168.20.0/24

Send traffic to:

10.0.0.2

# 29. Configure Static Route on R2

R2 also needs to know how to reach PC1's network.

On R2:

ip route 192.168.10.0 255.255.255.0 10.0.0.1

Meaning:

To reach:

192.168.10.0/24

Send traffic to:

10.0.0.1

Now both routers know how to reach the remote LAN.

# 30. Check Static Routes

On R1:

show ip route

You should see a static route similar to:

S    192.168.20.0/24 [1/0] via 10.0.0.2

S means Static route.

On R2:

show ip route

You should see:

S    192.168.10.0/24 [1/0] via 10.0.0.1

# 31. Test Connectivity

First test PC1 → R1:

ping 192.168.10.1

Then PC1 → R2:

ping 10.0.0.2

Then the important test:

ping 192.168.20.10

If everything is configured correctly:

PC1 → R1 → R2 → PC2

The ping should succeed.

# 32. Understand the Complete Packet Flow

PC1:

192.168.10.10

wants to communicate with:

192.168.20.10

The destination is on another network.

So PC1 sends the packet to its default gateway:

PC1

192.168.10.10

|

v

R1

192.168.10.1

|

v

10.0.0.2

R2

|

v

PC2

192.168.20.10

R1 checks its routing table and finds:

192.168.20.0/24 → 10.0.0.2

So R1 forwards the packet to R2.

R2 then forwards it to PC2.

# 33. Important Difference: VLAN vs Routing

Students should clearly understand this.

### VLAN

VLAN divides a switch into logical networks.

Switch

|

+--- VLAN 10

|

+--- VLAN 20

### Routing

Routing connects different IP networks.

Network A

|

Router

|

Network B

### Together

VLAN 10 ----\

Switch ---- Router ---- VLAN 20

VLAN 20 ----/

Routing is required when devices in different VLANs need to communicate.

# 34. Real-Time College Example

Suppose a college has:

VLAN 10 → Student Lab

VLAN 20 → Faculty

VLAN 30 → Admin

The switch separates these groups.

Switch

/     |     \

VLAN10 VLAN20 VLAN30

Lab    Faculty Admin

If the college wants Admin and Faculty networks to communicate with each other, a Layer 3 device must route traffic between those networks.

This is called inter-VLAN routing.

# 35. Common Problems During Practical

## Problem 1: Ping fails between PCs in same VLAN

Check:

1. IP address

2. Subnet mask

3. VLAN assignment

4. Cable

5. Switch port

Use:

show vlan brief

## Problem 2: Router interface is down

Run:

show ip interface brief

If required:

interface gigabitEthernet 0/0

no shutdown

## Problem 3: PC cannot reach another network

Check:

Default Gateway

Then check:

show ip route

## Problem 4: Static route is missing

On R1:

show ip route

Check whether the remote network exists.

Example:

192.168.20.0/24

If not, configure:

ip route 192.168.20.0 255.255.255.0 10.0.0.2

## Difference Between VLAN and Inter-VLAN

| VLAN | Inter-VLAN |
| --- | --- |
| VLAN stands for Virtual Local Area Network. | Inter-VLAN means communication between different VLANs. |
| It divides one physical network into multiple logical networks. | It allows different VLANs to communicate with each other. |
| Devices in the same VLAN can communicate with each other, subject to normal network configuration. | Devices in different VLANs need Layer 3 routing to communicate. |
| It improves network organization and security. | It enables communication between separate network segments. |
| VLAN works mainly at Layer 2 (Data Link Layer). | Inter-VLAN routing works at Layer 3 (Network Layer). |
| Example: VLAN 10 for the HR department and VLAN 20 for the IT department. | Example: A computer in HR (VLAN 10) communicates with a server in IT (VLAN 20). |

### Simple Example

VLAN — Separate Networks

VLAN 10

HR — PC1, PC2

VLAN 20

IT — PC3, PC4

Devices are in separate logical networks.

Inter-VLAN — Communication Between Networks

Layer 3 Switch or Router

Routes traffic between VLAN 10 and VLAN 20

# 36. Useful Cisco Commands

Students should practice these commands:

enable

configure terminal

show vlan brief

show ip interface brief

show ip route

show running-config

ping <IP-address>

traceroute <IP-address>

In Packet Tracer, the exact available commands can vary by device model and IOS version.

# 37. Day 3 Practical Task

## Part A – VLAN Task

Create this topology:

Switch

/    |    \

PC1   PC2   PC3

|     |     |

VLAN10 VLAN10 VLAN20

Configure:

VLAN 10 → LAB

VLAN 20 → ADMIN

Assign:

PC1 → 192.168.10.10

PC2 → 192.168.10.20

PC3 → 192.168.20.10

Test:

ping 192.168.10.20

Then:

ping 192.168.20.10

Students should explain why the first test and second test have different results in this basic Layer 2 setup.

# 38. Part B – Static Routing Task

Create:

PC1 ---- R1 ---- R2 ---- PC2

Use:

PC1 = 192.168.10.10

R1 LAN = 192.168.10.1

R1-R2:

R1 = 10.0.0.1

R2 = 10.0.0.2

R2 LAN = 192.168.20.1

PC2 = 192.168.20.10

Configure static routes:

### R1

ip route 192.168.20.0 255.255.255.0 10.0.0.2

### R2

ip route 192.168.10.0 255.255.255.0 10.0.0.1

Test from PC1:

ping 192.168.20.10

Then test from PC2:

ping 192.168.10.10

# 39. Student Documentation Task

Students should submit a small report containing:

### VLAN Configuration

VLAN ID:

VLAN Name:

Switch Port:

Connected Device:

IP Address:

### Static Routing

Router:

Destination Network:

Subnet Mask:

Next-Hop IP:

Example:

Router: R1

Destination: 192.168.20.0

Mask: 255.255.255.0

Next Hop: 10.0.0.2

### Verification

Include the output/results of:

show vlan brief

show ip interface brief

show ip route

ping

# Day 3 Learning Outcome

After this session, students should be able to:

- Explain what a VLAN is.

- Understand why VLANs are used.

- Create VLANs on a Cisco switch.

- Assign switch ports to VLANs.

- Understand access and trunk ports.

- Understand why different VLANs need routing to communicate.

- Explain what routing means.

- Explain static routing.

- Configure IP addresses on routers.

- Configure a static route.

- Verify routes using show ip route.

- Test connectivity using ping.

- Build a small routed network in Cisco Packet Tracer.