# Day 3: Introduction to IP Addressing

## 1. What is an IP Address?

An IP address is a logical address assigned to a network device.

It helps identify a device on an IP network and allows communication between devices.

### Simple Example

Suppose we have three computers:

PC 1 → 192.168.1.10

PC 2 → 192.168.1.20

PC 3 → 192.168.1.30

Each computer has a different IP address.

PC 1 ←→ PC 2 ←→ PC 3

The IP address helps the network know where data should be sent.

# 2. Why Do We Need IP Addresses?

Imagine a college has 100 computers.

If every computer has an IP address:

PC 1 → 192.168.1.10

PC 2 → 192.168.1.11

PC 3 → 192.168.1.12

...

PC 100 → 192.168.1.109

The network can identify the devices.

IP addresses are important for:

- Device identification

- Network communication

- Routing

- Internet communication

- Connecting different networks

# 3. IPv4

IPv4 stands for:

Internet Protocol Version 4

IPv4 is one of the most widely used IP addressing systems.

An IPv4 address contains 32 bits.

It is normally written using four decimal numbers separated by dots.

Example:

192.168.1.10

# 4. IPv4 Structure

IPv4 has 4 octets.

Example:

192 . 168 . 1 . 10

Each part is called an octet.

Each octet contains:

8 bits

Therefore:

8 + 8 + 8 + 8 = 32 bits

### Binary Example

192       168       1        10

↓         ↓         ↓        ↓

11000000  10101000  00000001 00001010

So:

IPv4 = 32 bits

# 5. IPv4 Range

Each IPv4 octet can contain values from:

0 to 255

Example of valid IPv4:

192.168.1.10

10.10.10.20

172.16.5.50

8.8.8.8

Invalid example:

192.168.1.300

Why?

Because an octet cannot be greater than 255.

# 6. IPv4 Address Example

Consider:

192.168.1.25

It contains:

Network part

+

Host part

The subnet mask tells us which portion represents the network and which portion represents the host.

For example:

IP Address:

192.168.1.25

Subnet Mask:

255.255.255.0

Here, the first three octets represent the network portion and the last octet is available for hosts in that subnet.

# 7. Private IPv4 Addresses

Private IP addresses are commonly used inside local networks.

Important private ranges:

### Range 1

10.0.0.0 – 10.255.255.255

### Range 2

172.16.0.0 – 172.31.255.255

### Range 3

192.168.0.0 – 192.168.255.255

Example:

192.168.1.10

is a private IP address.

These addresses are commonly used inside:

- Homes

- Colleges

- Offices

- Labs

- Companies

# 8. Public IP Address

A public IP address is an address used for communication over the public Internet.

Example:

Internet

|

Public IP

|

Router

|

Private Network

A home router commonly has a public-facing IP address and uses private IP addresses for devices inside the local network.

# 9. IPv4 Special Addresses

Students should know some important addresses.

### 127.0.0.1

This is the loopback address.

It refers to the local computer.

Computer → 127.0.0.1 → Same Computer

Test it:

ping 127.0.0.1

### 0.0.0.0

This can represent an unspecified address in different contexts.

For beginners, explain:

0.0.0.0 is not a normal host address assigned to a device. Its meaning depends on the networking context.

### 255.255.255.255

This is the IPv4 limited broadcast address.

# 10. IPv6

IPv6 stands for:

Internet Protocol Version 6

IPv6 was developed to provide a much larger address space than IPv4.

IPv6 uses:

128 bits

Example:

2001:db8:abcd:0012:0000:0000:0000:0001

# 11. IPv6 Structure

IPv6 is written using hexadecimal numbers.

Example:

2001:db8:abcd:0012:0000:0000:0000:0001

It contains 8 groups.

Each group contains up to 4 hexadecimal digits.

2001 : db8 : abcd : 0012 : 0000 : 0000 : 0000 : 0001

Each group represents:

16 bits

Therefore:

8 × 16 = 128 bits

# 12. IPv4 vs IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address size | 32 bits | 128 bits |
| Format | Decimal | Hexadecimal |
| Example | 192.168.1.10 | 2001:db8::1 |
| Separator | . | : |
| Address space | Smaller | Extremely large |
| Broadcast | Supported | No traditional broadcast |
| Common local address | 192.168.x.x | Depends on IPv6 addressing |
| Main purpose | IP communication | Larger modern IP address space |

# 13. Why Was IPv6 Needed?

IPv4 has a limited number of addresses.

The number of possible IPv4 addresses is:

2^32

which is approximately:

4.3 billion

Because billions of devices need network connectivity, IPv4 address space became limited.

IPv6 has:

2^128

possible addresses.

That is an extremely large address space.

# 14. IPv6 Address Simplification

IPv6 addresses can look long.

Example:

2001:0db8:0000:0000:0000:0000:0000:0001

We can remove leading zeros from each group.

2001:db8:0:0:0:0:0:1

We can then replace one continuous sequence of zero groups with ::.

2001:db8::1

### Important Rule

:: can be used only once in an IPv6 address.

Example:

2001:db8::1

is valid.

Do not write:

2001::db8::1

# 15. What is a Subnet?

A subnet is a smaller network created from a larger IP network.

Example:

Suppose a company has:

192.168.1.0/24

Instead of using one network for everyone, we can divide it into smaller networks.

For example:

Subnet 1

192.168.1.0/26

Subnet 2

192.168.1.64/26

Subnet 3

192.168.1.128/26

Subnet 4

192.168.1.192/26

This is called subnetting.

# 16. Why Do We Use Subnetting?

Subnetting helps us:

- Divide a large network.

- Organize devices.

- Reduce unnecessary broadcast traffic.

- Separate departments.

- Use IP addresses efficiently.

- Improve network management.

### Real-Life College Example

Suppose a college has:

Administration

50 devices

Computer Lab

100 devices

Faculty

30 devices

Library

20 devices

Instead of putting everything into one large network, the network can be divided into subnets.

College Network

|

+---- Administration

|

+---- Computer Lab

|

+---- Faculty

|

+---- Library

# 17. What is a Subnet Mask?

A subnet mask tells us which part of an IPv4 address represents the network and which part represents the host.

Example:

IP:

192.168.1.10

Subnet Mask:

255.255.255.0

The mask can be represented as:

/24

So:

192.168.1.10/24

means the first 24 bits are the network portion.

# 18. Common Subnet Masks

Students should remember these:

| CIDR | Subnet Mask | Total Addresses |
| --- | --- | --- |
| /8 | 255.0.0.0 | 16,777,216 |
| /16 | 255.255.0.0 | 65,536 |
| /24 | 255.255.255.0 | 256 |
| /25 | 255.255.255.128 | 128 |
| /26 | 255.255.255.192 | 64 |
| /27 | 255.255.255.224 | 32 |
| /28 | 255.255.255.240 | 16 |
| /29 | 255.255.255.248 | 8 |
| /30 | 255.255.255.252 | 4 |

For a normal IPv4 subnet, usable host addresses are commonly:

Total addresses - 2

because the network address and broadcast address are reserved.

For example:

/26

64 total addresses

64 - 2 = 62 usable hosts

# 19. What is CIDR?

CIDR stands for:

Classless Inter-Domain Routing

CIDR represents the network prefix using /number.

Example:

192.168.1.0/24

Here:

/24

means:

24 bits = Network portion

8 bits = Host portion

Because IPv4 has 32 bits:

32 - 24 = 8

# 20. CIDR Examples

### /24

Network bits = 24

Host bits = 8

Total addresses:

2^8 = 256

Usable:

256 - 2 = 254

### /25

Network bits = 25

Host bits = 7

Total:

2^7 = 128

Usable:

126

### /26

Network bits = 26

Host bits = 6

Total:

2^6 = 64

Usable:

62

### /27

Network bits = 27

Host bits = 5

Total:

2^5 = 32

Usable:

30

# 21. Subnetting Formula

For basic IPv4 subnetting:

### Total addresses

2^host_bits

### Usable hosts

2^host_bits - 2

### Number of subnets

If you borrow n host bits:

2^n

subnets are created from the original network.

# 22. Subnetting Example 1

Given:

192.168.1.0/24

Suppose we change it to:

/26

### Step 1: Find host bits

32 - 26 = 6

### Step 2: Total addresses

2^6 = 64

### Step 3: Usable hosts

64 - 2 = 62

### Step 4: Find subnet ranges

Block size:

256 - 192 = 64

Therefore:

Subnet 1:

192.168.1.0 - 192.168.1.63

Subnet 2:

192.168.1.64 - 192.168.1.127

Subnet 3:

192.168.1.128 - 192.168.1.191

Subnet 4:

192.168.1.192 - 192.168.1.255

# 23. Complete /26 Table

| Subnet | Network | First Usable | Last Usable | Broadcast |
| --- | --- | --- | --- | --- |
| 1 | 192.168.1.0 | 192.168.1.1 | 192.168.1.62 | 192.168.1.63 |
| 2 | 192.168.1.64 | 192.168.1.65 | 192.168.1.126 | 192.168.1.127 |
| 3 | 192.168.1.128 | 192.168.1.129 | 192.168.1.190 | 192.168.1.191 |
| 4 | 192.168.1.192 | 192.168.1.193 | 192.168.1.254 | 192.168.1.255 |

Each subnet has:

64 total addresses

62 usable host addresses

# 24. Subnetting Example 2

Given:

192.168.10.0/27

### Step 1

Host bits:

32 - 27 = 5

### Step 2

Total addresses:

2^5 = 32

### Step 3

Usable hosts:

32 - 2 = 30

### Step 4

Subnet mask:

255.255.255.224

### Step 5

Block size:

256 - 224 = 32

Subnets:

192.168.10.0

192.168.10.32

192.168.10.64

192.168.10.96

192.168.10.128

192.168.10.160

192.168.10.192

192.168.10.224

# 25. Example: Find Network and Broadcast

Given:

IP = 192.168.10.70/27

Subnet ranges are:

0 - 31

32 - 63

64 - 95

96 - 127

...

70 belongs to:

64 - 95

Therefore:

Network Address:

192.168.10.64

First Usable:

192.168.10.65

Last Usable:

192.168.10.94

Broadcast:

192.168.10.95

Usable Hosts:

30

# 26. Another Important Example

Given:

10.10.10.140/27

Subnet ranges:

10.10.10.0 - 31

10.10.10.32 - 63

10.10.10.64 - 95

10.10.10.96 - 127

10.10.10.128 - 159

10.10.10.160 - 191

...

140 belongs to:

128 - 159

Therefore:

Network:

10.10.10.128

First Usable:

10.10.10.129

Last Usable:

10.10.10.158

Broadcast:

10.10.10.159

Usable Hosts:

30

# Practical Part

# Practical 1: Check Current IP

On Windows, open:

Command Prompt

Run:

ipconfig

Students should record:

IPv4 Address

Subnet Mask

Default Gateway

Example:

IPv4 Address : 192.168.1.20

Subnet Mask  : 255.255.255.0

Gateway      : 192.168.1.1

# Practical 2: Find Complete Network Information

Run:

ipconfig /all

Students should find:

Physical Address

DHCP Enabled

IPv4 Address

Subnet Mask

Default Gateway

DNS Servers

Explain what each field means.

# Practical 3: Find Your IP Address Using Settings

Students can also check through Windows:

Settings

↓

Network & Internet

↓

Wi-Fi / Ethernet

↓

Properties

They can compare the information with:

ipconfig

# Practical 4: Assign Static IPv4 Address

This is the main practical.

### Important

Do this only on a controlled lab network where you know the correct IP range. Do not randomly change the IP of a computer connected to an important network.

Go to:

Control Panel

↓

Network and Internet

↓

Network and Sharing Center

↓

Change adapter settings

Right-click:

Ethernet / Wi-Fi

Select:

Properties

Select:

Internet Protocol Version 4 (TCP/IPv4)

Click:

Properties

Select:

Use the following IP address

For a simple isolated lab network, for example:

### Computer 1

IP Address:

192.168.10.10

Subnet Mask:

255.255.255.0

### Computer 2

IP Address:

192.168.10.20

Subnet Mask:

255.255.255.0

If the lab is directly connected without a router, a default gateway is not necessarily required for PC-to-PC communication.

# Practical 5: Test Connectivity Between Two PCs

Suppose:

PC 1:

192.168.10.10

PC 2:

192.168.10.20

From PC 1:

ping 192.168.10.20

If connectivity is working, students may see:

Reply from 192.168.10.20

Then from PC 2:

ping 192.168.10.10

This demonstrates basic IP communication.

# Practical 6: Troubleshoot If Ping Fails

If ping does not work, students should check step by step:

### Step 1

Check IP:

ipconfig

### Step 2

Check subnet mask.

Both PCs should be in the same subnet for this simple direct/isolated example.

### Step 3

Check cable or Wi-Fi connection.

### Step 4

Check whether Windows Firewall is blocking ICMP.

### Step 5

Try:

ping 127.0.0.1

This checks the local TCP/IP stack.

### Step 6

Try pinging the other PC again.

This is also a good connection with the OSI/TCP-IP troubleshooting concept from Day 1 and Day 2.

# Practical 7: Static IP Assignment Activity

Give students this setup:

PC 1 → 192.168.50.10

PC 2 → 192.168.50.20

PC 3 → 192.168.50.30

PC 4 → 192.168.50.40

Subnet mask:

255.255.255.0

Students should:

- Assign the IP.

- Run ipconfig.

- Verify the IP.

- Ping another computer.

- Record the result.

# Practical 8: Use CIDR With Static IP

Explain that:

255.255.255.0

is the same as:

/24

Therefore:

192.168.50.10

255.255.255.0

can be written as:

192.168.50.10/24

Students should understand that both represent the same network prefix information.

# Practical 9: Subnetting Exercise

Give students:

Network:

192.168.1.0/24

Ask them to create:

4 subnets

They should find:

New CIDR

Subnet Mask

Network Address

First Usable

Last Usable

Broadcast

Usable Hosts

Expected new prefix:

/26

# Practical 10: Subnetting With an IP

Give:

IP:

192.168.1.70/26

Ask students to find:

1. Subnet Mask

2. Network Address

3. First Usable IP

4. Last Usable IP

5. Broadcast Address

6. Total Addresses

7. Usable Hosts

# Practical 11: More Subnetting Questions

### Question 1

192.168.1.100/27

Find:

- Network

- First usable

- Last usable

- Broadcast

- Total addresses

- Usable hosts

### Question 2

192.168.5.145/28

Find:

- Network

- First usable

- Last usable

- Broadcast

- Total addresses

- Usable hosts

### Question 3

10.10.10.140/27

Find:

- Network

- First usable

- Last usable

- Broadcast

- Total addresses

- Usable hosts

### Question 4

172.16.20.200/28

Find:

- Network

- First usable

- Last usable

- Broadcast

- Total addresses

- Usable hosts

# IPv6 Practical Introduction

For Day 3, don't go deep into IPv6 subnetting yet. First make students comfortable with IPv6 format.

Run:

ipconfig

Look for:

IPv6 Address

Students may see a long IPv6 address.

They can identify:

- IPv6 address

- Link-local IPv6 address

- IPv4 address

Explain that Windows may show addresses beginning with:

fe80::

These are commonly IPv6 link-local addresses.

# IPv6 Basic Practice

Give students these addresses and ask them to simplify them.

### Example 1

2001:0db8:0000:0000:0000:0000:0000:0001

Answer:

2001:db8::1

### Example 2

2001:0db8:0000:0000:0000:0000:abcd:0010

Answer:

2001:db8::abcd:10

### Example 3

2001:0db8:0000:0000:1234:0000:0000:0050

Answer:

2001:db8::1234:0:0:50

# Day 3 Final Practical

Give students one complete practical exercise.

## Part A – Find Current Network Details

Run:

ipconfig /all

Record:

IPv4 Address

IPv6 Address

Subnet Mask

Default Gateway

MAC Address

DNS Server

## Part B – Connectivity

Run:

ping 127.0.0.1

Then:

ping 8.8.8.8

Then:

ping google.com

Ask students to explain the difference between the three tests.

## Part C – Static IP

On a controlled lab network, assign:

PC 1:

192.168.50.10/24

PC 2:

192.168.50.20/24

Test:

ping 192.168.50.20

## Part D – Subnetting

Solve:

192.168.10.70/27

Find:

Subnet Mask

Network Address

First Usable

Last Usable

Broadcast

Total Addresses

Usable Hosts

## Part E – IPv6

Simplify:

2001:0db8:0000:0000:0000:0000:0000:0001

# Day 3 Quick Revision

Students should remember:

IPv4

↓

32 bits

↓

Decimal

↓

192.168.1.10

IPv6

↓

128 bits

↓

Hexadecimal

↓

2001:db8::1

CIDR

↓

/24 /25 /26 /27 /28

Subnet Mask

↓

255.255.255.0

255.255.255.128

255.255.255.192

255.255.255.224

255.255.255.240

And the basic subnetting calculation:

Host Bits = 32 - CIDR

Total Addresses = 2^Host Bits

Usable Hosts = Total Addresses - 2

For example:

192.168.1.70/27

32 - 27 = 5 host bits

2^5 = 32 total addresses

32 - 2 = 30 usable hosts

Subnet Mask = 255.255.255.224

Network = 192.168.1.64

First = 192.168.1.65

Last = 192.168.1.94

Broadcast = 192.168.1.95

This gives students the complete flow from IP address → CIDR → subnet mask → network → first/last usable → broadcast → usable hosts, which is the foundation they will need for the next networking practicals.