# Day 5: Review and Practice

## Topic: Create and Configure a Small Network in Cisco Packet Tracer

### 1. Day 5 Objective

Today students will revise the main networking concepts learned so far:

- Network Topology

- Switch

- Router

- IP Address

- Subnet Mask

- Default Gateway

- VLAN

- Static Routing

- Firewall basics

- Ping and basic troubleshooting

- Cisco Packet Tracer

The main practical activity is to create a small physical network topology in Cisco Packet Tracer, configure IP addresses, and test communication.

# Part 1: Small Network Topology

For today's practical, create this network:

PC1 --------\

PC2 ---------- Switch1 -------- Router1

PC3 --------/                     |

|

Internet

For a simple LAN demonstration, students can first create:

PC1

|

PC2 ---- Switch

|

PC3

Then extend it with a router:

PC1 ----\

PC2 ----- Switch ---- Router

PC3 ----/

### Devices Required

- 3 PCs

- 1 Switch

- 1 Router

- Copper Straight-Through cables

Suggested Cisco devices:

- Switch: 2960

- Router: 1941

# Part 2: IP Address Plan

Use the following network:

Network: 192.168.10.0/24

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC2 | 192.168.10.20 | 255.255.255.0 | 192.168.10.1 |
| PC3 | 192.168.10.30 | 255.255.255.0 | 192.168.10.1 |
| Router | 192.168.10.1 | 255.255.255.0 | - |

### Why use the same network?

All three PCs are in:

192.168.10.0/24

So they can communicate with each other through the switch.

The router IP:

192.168.10.1

will act as the default gateway.

# Part 3: Create the Topology

### Step 1: Open Cisco Packet Tracer

Open Cisco Packet Tracer.

### Step 2: Add Devices

Add:

3 × PC

1 × Switch

1 × Router

Arrange them like:

PC1 --------\

\

PC2 ---------- Switch -------- Router

/

PC3 --------/

### Step 3: Connect Devices

Use:

Connections → Copper Straight-Through

Connect:

PC1 FastEthernet0 → Switch FastEthernet0/1

PC2 FastEthernet0 → Switch FastEthernet0/2

PC3 FastEthernet0 → Switch FastEthernet0/3

Switch FastEthernet0/24 → Router GigabitEthernet0/0

Wait until the links become active.

# Part 4: Configure the Router

Click the router.

Go to:

CLI

Enter:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

Now check the interface:

show ip interface brief

You should see something similar to:

GigabitEthernet0/0    192.168.10.1    YES    manual    up    up

### Important

no shutdown is required to activate the router interface.

# Part 5: Configure PC1

Click:

PC1 → Desktop → IP Configuration

Enter:

IP Address:      192.168.10.10

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

# Part 6: Configure PC2

Enter:

IP Address:      192.168.10.20

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

# Part 7: Configure PC3

Enter:

IP Address:      192.168.10.30

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

# Part 8: Test the Network

Open:

PC1 → Desktop → Command Prompt

Run:

ipconfig

Check whether the IP address is correct.

Then:

ping 192.168.10.20

PC1 should communicate with PC2.

Test PC3:

ping 192.168.10.30

Test the router:

ping 192.168.10.1

If everything is configured correctly, you should receive:

Reply from 192.168.10.20

Reply from 192.168.10.30

Reply from 192.168.10.1

# Part 9: Understand the Communication

Suppose:

PC1 → PC2

PC1 has:

IP = 192.168.10.10

PC2 has:

IP = 192.168.10.20

Both are in the same network:

192.168.10.0/24

So the communication happens through the switch.

PC1

|

Switch

|

PC2

The router is not required for communication between these two PCs.

## PC1 → Another Network

Suppose PC1 wants to communicate with:

192.168.20.10

This is a different network.

PC1 sends the traffic to its default gateway:

PC1 → Switch → Router → Other Network

This is where the router becomes important.

# Part 10: Review VLAN Concept

Now students can also review VLANs from Day 3.

For example:

VLAN 10 = LAB

VLAN 20 = ADMIN

A switch can have:

PC1 → VLAN 10

PC2 → VLAN 10

PC3 → VLAN 20

PC1 and PC2 can communicate at Layer 2, but PC1 and PC3 need inter-VLAN routing to communicate.

Useful command:

show vlan brief

This helps students remember the previous day's practical.

# Part 11: Troubleshooting Practice

Now intentionally create some problems and ask students to find them.

### Problem 1: Wrong IP

Change PC2:

192.168.10.20

to:

192.168.20.20

Then from PC1:

ping 192.168.20.20

Ask students:

Why is communication not working?

Expected concept:

PC2 is now in a different network.

### Problem 2: Wrong Gateway

Change PC1 gateway to:

192.168.10.100

Ask:

Can PC1 communicate with the router?

Students should identify the incorrect default gateway.

### Problem 3: Router Interface Shutdown

On the router:

enable

configure terminal

interface gigabitEthernet 0/0

shutdown

Now test:

ping 192.168.10.1

Students should identify that the router interface is administratively down.

Fix:

no shutdown

### Problem 4: Wrong Cable

Disconnect one PC from the switch.

Ask students to check:

Is the problem with IP configuration or physical connectivity?

This reviews the Physical Layer concept from Day 1.

# Part 12: Troubleshooting Flow

Give students this simple troubleshooting sequence:

1. Check cable

↓

2. Check link status

↓

3. Check IP address

↓

4. Check subnet mask

↓

5. Check default gateway

↓

6. Ping own IP

↓

7. Ping another PC

↓

8. Ping router

↓

9. Check VLAN

↓

10. Check routing

Useful commands:

ipconfig

ping 127.0.0.1

ping <own-IP>

ping <other-PC-IP>

ping <gateway-IP>

On router:

show ip interface brief

show ip route

On switch:

show vlan brief

# Part 13: Real-Life Example

Explain this example to students:

Imagine a college network.

LAB PCs

|

↓

Switch

|

↓

Router

|

↓

Internet

Students in the computer lab use PCs connected to a switch.

The switch connects all PCs in the local network.

The router connects the college network to other networks or the Internet.

If one PC cannot access another PC:

PC → Switch → PC

If the PC cannot access the Internet:

PC → Switch → Router → Internet

So the troubleshooting process should check each part one by one.

# Part 14: Student Task – Document a Small Network

Students must prepare documentation for the network they created.

### Documentation Format

1. Network Name

Small College Network

2. Devices Used

3 PCs

1 Switch

1 Router

3. Network Address

192.168.10.0/24

4. IP Address Table

PC1 → 192.168.10.10

PC2 → 192.168.10.20

PC3 → 192.168.10.30

Router → 192.168.10.1

5. Connections

Document which device is connected to which switch port.

Example:

PC1 → Fa0/1

PC2 → Fa0/2

PC3 → Fa0/3

Router → Fa0/24

6. Router Configuration

Write the commands used to configure the router.

7. Testing

Document:

PC1 → PC2 = Success/Fail

PC1 → PC3 = Success/Fail

PC1 → Router = Success/Fail

8. Troubleshooting

If there was an error, write:

Problem:

Wrong IP address on PC2

Solution:

Changed PC2 IP to 192.168.10.20

# Final Practical Challenge

Give students this task without showing the solution immediately:

### Create this network:

PC1

|

|

PC2 ------ Switch ------ Router

|

|

PC3

Requirements:

- Use 3 PCs.

- Use 1 switch.

- Use 1 router.

- Configure all IP addresses.

- Configure the router interface.

- Set the default gateway on all PCs.

- Test PC-to-PC communication.

- Test PC-to-router communication.

- Use ipconfig and ping.

- Check the router using show ip interface brief.

- Intentionally create one network problem.

- Find and fix the problem.

- Document all configuration steps.

### Expected final result

PC1  ✓

PC2  ✓ ---- Switch ---- Router ✓

PC3  ✓

Students should be able to build, configure, test, troubleshoot, and document a complete small network by the end of this practical.