# Day 5: Network Testing Tools

## 1. Theory – Network Testing Tools

Network testing tools help us check whether a device, network, or website is working correctly.

Today we will mainly use:

- Ping

- Tracert / Traceroute

- Nslookup

These tools are available from the Windows Command Prompt (CMD).

# 2. Ping

## What is Ping?

ping is used to check whether one device can communicate with another device over a network.

It sends a small test packet to the destination and waits for a reply.

### Real-life example

Suppose:

- PC 1 = 192.168.10.10

- PC 2 = 192.168.10.20

From PC 1, we can run:

ping 192.168.10.20

If PC 2 replies, we know that basic network communication is working.

### Example output

Pinging 192.168.10.20 with 32 bytes of data:

Reply from 192.168.10.20: bytes=32 time<1ms TTL=128

Reply from 192.168.10.20: bytes=32 time<1ms TTL=128

Reply from 192.168.10.20: bytes=32 time<1ms TTL=128

Reply from 192.168.10.20: bytes=32 time<1ms TTL=128

This means the destination is reachable.

## Important Ping Terms

### Reply from

The destination responded successfully.

### Request timed out

The destination did not reply within the expected time.

Possible reasons:

- Device is turned off.

- Wrong IP address.

- Network cable problem.

- Wi-Fi disconnected.

- Firewall blocking ICMP.

- Wrong subnet configuration.

- Routing problem.

### Destination host unreachable

The computer cannot find a path to the destination.

Possible reasons:

- Wrong IP/subnet configuration.

- Network adapter problem.

- Gateway/router problem.

- Destination network is unavailable.

## Useful Ping Commands

### Test your own computer

ping 127.0.0.1

127.0.0.1 is the IPv4 loopback address.

It checks whether the local TCP/IP stack is working.

### Test your own IP

First check your IP:

ipconfig

Suppose your IP is:

192.168.10.10

Then:

ping 192.168.10.10

### Test another computer in LAN

ping 192.168.10.20

### Test the default gateway

First:

ipconfig

Suppose the gateway is:

192.168.10.1

Then:

ping 192.168.10.1

### Test a website

ping google.com

This checks whether the destination can be reached using its hostname.

Note: Some websites may not respond to ping even when the website itself is working.

# 3. Tracert / Traceroute

## What is Traceroute?

Traceroute shows the path taken by packets from your computer to a destination.

In Windows, the command is:

tracert

In Linux:

traceroute

### Example

tracert google.com

You may see something like:

Tracing route to google.com

1    192.168.10.1

2    10.10.0.1

3    172.16.5.1

4    203.0.113.1

5    ...

Each line represents a hop.

A hop can be a router or another Layer 3 device along the path.

## What is a Hop?

A hop is one network device that forwards the packet toward its destination.

Example:

Your PC

|

v

Wi-Fi Router

|

v

ISP Router

|

v

Internet Router

|

v

Destination

The routers between the source and destination are called hops.

## Why do we use Tracert?

Tracert is useful when:

- Ping is failing.

- Internet is slow.

- We want to know where the connection is going.

- We want to find where a routing problem may occur.

- We want to understand the path to a remote server.

### Example

Suppose:

ping example.com

does not work.

We can try:

tracert example.com

If the packets stop at a particular point, it may help us identify where the problem is occurring.

## Important Point

You may see:

*

*

*

in a tracert result.

This does not always mean the network is broken.

Some routers do not respond to traceroute probes or may intentionally limit these responses.

# 4. Nslookup

## What is Nslookup?

nslookup is used to check DNS information.

DNS means:

Domain Name System

DNS converts a domain name into an IP address.

Example:

google.com

↓

IP address

Humans remember names such as:

google.com

Computers communicate using IP addresses.

## Basic Command

nslookup google.com

Example:

Server:  router.local

Address: 192.168.10.1

Non-authoritative answer:

Name:    google.com

Address: 142.250.x.x

The exact IP address may be different.

## Why is Nslookup useful?

It helps us check:

- Is DNS working?

- Which DNS server is being used?

- What IP address belongs to a domain?

- Is the domain resolving correctly?

- What are the DNS records?

## Find DNS Server

Run:

ipconfig /all

Look for:

DNS Servers

Example:

DNS Servers . . . . . . . : 192.168.10.1

## Nslookup with a specific DNS server

For example:

nslookup google.com 8.8.8.8

This asks the DNS server 8.8.8.8 to resolve the domain.

# 5. Ping vs Tracert vs Nslookup

| Tool | Main Purpose | Example |
| --- | --- | --- |
| Ping | Check connectivity | ping 192.168.10.20 |
| Tracert | Find network path | tracert google.com |
| Nslookup | Check DNS resolution | nslookup google.com |

### Easy way to remember

PING

"Can I reach it?"

TRACERT

"How do I reach it?"

NSLOOKUP

"What IP address does this name have?"

# 6. Real-Time Troubleshooting Example

Suppose a student says:

"My computer is connected to Wi-Fi, but I cannot open websites."

Instead of immediately changing settings, we can troubleshoot step-by-step.

### Step 1: Check IP configuration

ipconfig

Check:

- IPv4 Address

- Subnet Mask

- Default Gateway

### Step 2: Test your network stack

ping 127.0.0.1

If this fails, there may be a local TCP/IP problem.

### Step 3: Test your computer's IP

Suppose:

IPv4 Address: 192.168.10.20

Run:

ping 192.168.10.20

### Step 4: Test the gateway

Suppose:

Default Gateway: 192.168.10.1

Run:

ping 192.168.10.1

If this fails, check the local LAN/Wi-Fi connection, IP configuration, or gateway.

### Step 5: Test an Internet IP

For a controlled lab demonstration, you can test a known reachable public IP:

ping 8.8.8.8

If this works but:

ping google.com

does not work, DNS may be the problem.

### Step 6: Check DNS

nslookup google.com

If DNS resolution fails, investigate the DNS configuration.

### Step 7: Check the route

tracert google.com

This helps understand where traffic is going and where responses may stop.

# 7. Practical – 30 Minutes

## Practical Objective

Students will use:

- ping

- tracert

- nslookup

to test connectivity and basic network functionality.

## Practical 1: Check Local TCP/IP

Open CMD:

ping 127.0.0.1

### Observe:

- Reply

- Time

- Packet loss

Students should record the result.

# Practical 2: Check Own IP

Run:

ipconfig

Find:

IPv4 Address

Subnet Mask

Default Gateway

DNS Server

Then ping your own IP.

Example:

ping 192.168.10.10

# Practical 3: Test LAN Connectivity

If two PCs are connected to the same lab network:

### PC 1

IP: 192.168.10.10

Mask: 255.255.255.0

### PC 2

IP: 192.168.10.20

Mask: 255.255.255.0

From PC 1:

ping 192.168.10.20

From PC 2:

ping 192.168.10.10

### Expected result

Both computers should receive replies if the LAN and firewall configuration allow ICMP.

# Practical 4: Test Default Gateway

Run:

ipconfig

Find:

Default Gateway

Example:

192.168.10.1

Then:

ping 192.168.10.1

Record the result.

# Practical 5: Use Nslookup

Run:

nslookup google.com

Students should identify:

- DNS server

- Domain name

- Returned IP address

Try another domain:

nslookup microsoft.com

# Practical 6: Use Tracert

Run:

tracert google.com

Ask students to observe:

- Number of hops

- IP addresses

- Response times

- Any * entries

# Practical 7: Compare Results

Students should understand:

### Ping

ping google.com

Question:

Can I reach the destination?

### Tracert

tracert google.com

Question:

What path does the traffic take?

### Nslookup

nslookup google.com

Question:

What IP address does the domain name resolve to?

# 8. Practical Troubleshooting Flow

Give students this simple troubleshooting flow:

Start

↓

ipconfig

↓

Check IPv4 + Gateway

↓

ping 127.0.0.1

↓

ping own IP

↓

ping Gateway

↓

ping another LAN PC

↓

nslookup google.com

↓

tracert google.com

↓

Record Results

This gives students a basic real-world network troubleshooting process.

# 9. Task – Network Testing Report

Students have to prepare a small report based on the practical.

## Report Format

### 1. System Information

Computer Name:

IPv4 Address:

Subnet Mask:

Default Gateway:

DNS Server:

### 2. Ping Test

Command:

ping 127.0.0.1

Result:

Successful / Failed

Command:

ping <LAN PC IP>

Result:

Successful / Failed

Command:

ping <Gateway IP>

Result:

Successful / Failed

### 3. Nslookup Test

Command:

nslookup google.com

Record:

DNS Server:

Domain:

Resolved IP:

Result:

### 4. Tracert Test

Command:

tracert google.com

Record:

Number of visible hops:

First hop:

Some intermediate hops:

Final destination:

### 5. Final Observation

Students should write:

Ping was used to test connectivity.

Tracert was used to understand the path to the destination.

Nslookup was used to check DNS name resolution.

The tests helped us identify basic LAN, Internet and DNS-related network problems.

# 10. Day 5 Final Practical Challenge

Give students this real-life situation:

A computer is connected to the LAN, but the user says that websites are not opening. Find the possible problem using CMD tools.

Students should use:

ipconfig

then:

ping 127.0.0.1

then:

ping <own-IP>

then:

ping <gateway-IP>

then:

ping 8.8.8.8

then:

nslookup google.com

then:

tracert google.com

They must record each result and explain which test passed, which failed, and what that result tells them.

## Day 5 Learning Outcome

After this practical, students should be able to:

- Use ping to test connectivity.

- Use tracert to understand network paths.

- Use nslookup to troubleshoot DNS.

- Check IP, gateway and DNS configuration.

- Test communication between LAN computers.

- Read basic CMD network-test results.

- Prepare a simple network testing report.

- Use multiple tools together for basic network troubleshooting.