# Day 6: Introduction to Network Devices

## 1. Introduction

Network devices are hardware devices used to connect computers, phones, servers, printers, and other devices.

A simple network may look like this:

Internet

|

Router

|

+-----+-----+

|           |

Switch     Access Point

|           |

+-----+-----+    Wi-Fi

|     |     |      |

PC1   PC2  Printer  Laptop

Different devices perform different jobs.

Today we will learn:

- Hub

- Switch

- Router

- Firewall

- Access Point

# 2. Hub

## What is a Hub?

A hub is a basic networking device used to connect multiple devices in a LAN.

When a hub receives data from one port, it sends that data to all other ports.

### Example

Suppose:

PC1

|

Hub ---- PC2

|

PC3

|

PC4

If PC1 sends data to PC3, the hub sends the data to PC2, PC3, and PC4.

PC3 accepts the data, while the other devices ignore it.

## Hub Features

- Works mainly at OSI Layer 1 – Physical Layer

- Uses physical connections.

- Does not understand MAC addresses.

- Sends incoming data to all connected ports.

- Creates more unnecessary network traffic.

- Mostly replaced by switches in modern networks.

### Real-life example

Think of a hub like a person announcing:

"I have a message for PC3!"

Everyone in the room hears the message, but only PC3 needs to respond.

# 3. Switch

## What is a Switch?

A switch connects multiple devices in a LAN.

Unlike a hub, a switch can learn the MAC address of connected devices and forward Ethernet frames toward the appropriate port.

### Example

Switch

/    |     \

PC1   PC2   Printer

Suppose PC1 wants to send data to PC2.

The switch checks its MAC address table and forwards the frame toward PC2's port.

## Switch Features

- Mainly works at OSI Layer 2 – Data Link Layer.

- Uses MAC addresses.

- Maintains a MAC address table.

- Connects computers, printers, servers, and other LAN devices.

- Reduces unnecessary traffic compared with a hub.

- Modern switches can also perform Layer 3 routing functions.

## Common Switch Ports

You may see:

Port 1

Port 2

Port 3

...

Port 24

Common switch types:

- 5-port switch

- 8-port switch

- 16-port switch

- 24-port switch

- 48-port switch

Some switches also have:

- Gigabit Ethernet ports

- SFP/SFP+ ports

- PoE ports

# 4. Router

## What is a Router?

A router connects different networks.

For example:

LAN

192.168.1.0/24

|

Router

|

Internet

The router decides where IP packets should go.

## Real-Life Example

Suppose your office network is:

192.168.10.0/24

The Internet is a different network.

The router acts as a connection between your LAN and the Internet.

Office PC

192.168.10.20

|

v

Router

192.168.10.1

|

v

Internet

The router's LAN-side IP is commonly configured as the default gateway for devices on that LAN.

## Router Features

- Works primarily at OSI Layer 3 – Network Layer.

- Uses IP addresses.

- Connects different networks.

- Performs routing.

- Provides a default gateway for hosts on a LAN.

- Many home routers also provide NAT, DHCP, firewall functions, and Wi-Fi.

## Common Router Ports

Depending on the router, you may see:

### WAN / Internet Port

Used to connect toward an ISP or another upstream network.

### LAN Ports

Used to connect local devices.

Example:

Router

+-------+-------+

|       |       |

WAN    LAN 1   LAN 2

|       |       |

Internet   PC     Switch

# 5. Firewall

## What is a Firewall?

A firewall controls network traffic based on security rules.

It can allow or block traffic depending on factors such as:

- Source IP

- Destination IP

- Port

- Protocol

- Connection state

- Application, depending on the firewall

### Simple example

Suppose an office wants to allow web traffic but block a particular service.

A firewall can use rules to control that traffic.

Internet

|

v

Firewall

|

v

Office Network

## Firewall Features

- Helps protect networks from unauthorized traffic.

- Can allow or block connections.

- Uses security rules/policies.

- Can inspect network traffic.

- Can exist as hardware, software, or a cloud service.

- Enterprise firewalls may provide VPN, NAT, intrusion prevention, logging, and other features.

## Simple Example

Rule:

Allow HTTPS

Port: 443

Traffic using HTTPS may be allowed according to the firewall policy.

Another rule might block a particular destination or port.

# 6. Access Point

## What is an Access Point?

An Access Point (AP) provides wireless network access to devices such as:

- Laptop

- Smartphone

- Tablet

- Smart TV

- Wireless printer

Example:

Switch

|

|

Access Point

/     |      \

Laptop Phone   Tablet

\     |      /

Wi-Fi

The AP provides Wi-Fi connectivity to wireless clients.

## Access Point Features

- Provides wireless network access.

- Uses Wi-Fi technology.

- Connects wireless clients to a wired LAN.

- Has an SSID (Wi-Fi network name).

- Supports wireless security such as WPA2/WPA3, depending on the device.

- Enterprise APs may support multiple SSIDs, VLANs, roaming, and centralized management.

# 7. Hub vs Switch

This is an important difference.

| Feature | Hub | Switch |
| --- | --- | --- |
| Main OSI layer | Layer 1 | Layer 2 |
| Uses MAC address | No | Yes |
| Sends data | To all ports | Toward the appropriate port |
| Traffic | More unnecessary traffic | More efficient |
| Intelligence | Very basic | More intelligent |
| Modern use | Rare | Very common |

### Easy Example

Hub:

PC1 → Hub → PC2

→ PC3

→ PC4

Everyone receives the signal.

Switch:

PC1 → Switch → PC2

The switch forwards the frame based on its MAC address table.

# 8. Switch vs Router

| Feature | Switch | Router |
| --- | --- | --- |
| Main OSI layer | Layer 2 | Layer 3 |
| Main address | MAC | IP |
| Main purpose | Connect devices in a LAN | Connect different networks |
| Example | PC to printer | LAN to Internet |
| Routing | Usually not on basic Layer 2 switches | Yes |
| Default gateway | No | Yes |

### Easy Example

PC1 ----\

PC2 ----- Switch ---- Router ---- Internet

PC3 ----/

The switch connects the local devices.

The router connects the local network to another network.

# 9. Router vs Firewall

These devices can sometimes exist in the same physical appliance, especially in home networking equipment, but their primary purposes are different.

### Router

Main question:

"Where should this IP packet go?"

### Firewall

Main question:

"Should this traffic be allowed according to the security policy?"

Example:

Internet

|

Router

|

Firewall

|

Switch

|

PCs

In many modern networks, routing and firewall functions can be performed by the same appliance.

# 10. Access Point vs Router Wi-Fi

This can confuse beginners.

### Home Wi-Fi Router

A typical home Wi-Fi router may combine:

Router

+ Switch

+ Access Point

+ Firewall

+ NAT

+ DHCP

into one device.

### Dedicated Access Point

A dedicated AP mainly provides wireless connectivity.

Example:

Internet

|

Router/Firewall

|

Switch

|

Access Point

|

Wi-Fi Devices

# 11. Important Ports to Identify

During the practical, students should look at the physical ports.

## Ethernet Port

Usually looks like:

RJ45 Ethernet

Used for network cables.

Common labels:

LAN

WAN

Ethernet

GE

10/100/1000

## Console Port

Some routers and switches have a:

Console

port.

It is used for device configuration and management.

## USB Port

Some network devices include USB ports for specific functions such as:

- Configuration

- Storage

- Modem support

- Firmware/service functions

The exact use depends on the device.

## SFP Port

Enterprise switches and routers may have:

SFP

SFP+

SFP28

ports.

These can be used with compatible optical or copper transceiver modules.

# 12. Practical – Identify Network Devices

## Practical Objective

Students should physically identify network devices and their ports.

Use available lab devices such as:

- Switch

- Router

- Old hub, if available

- Firewall appliance, if available

- Wireless Access Point

Do not disconnect or change configuration on a live network without permission.

# Practical 1: Identify a Switch

Ask students to observe the switch.

Find:

1. Power port

2. Ethernet ports

3. Port numbers

4. Link/activity LEDs

5. Console port, if available

6. SFP ports, if available

Students should identify which ports currently have network cables connected.

# Practical 2: Identify a Router

Look for:

1. WAN/Internet port

2. LAN ports

3. Power port

4. Reset button

5. Antennas, if present

6. Status LEDs

Ask students:

Which port connects toward the Internet/upstream network?

Answer should be based on the actual device's labels and setup.

# Practical 3: Identify an Access Point

Check:

1. Ethernet port

2. Power connection

3. Antennas, if present

4. Status LEDs

5. Reset button

6. Device label/model

Check the available Wi-Fi network name (SSID) from a permitted client device.

# Practical 4: Identify Firewall

If a firewall appliance is available, identify:

WAN interface

LAN interface

Management interface

Console port

Power

Status LEDs

Explain that the actual number and naming of interfaces depends on the firewall model.

# Practical 5: Understand a Complete Network Setup

Students can observe this setup:

Internet

|

|

Router

|

Firewall

|

Switch

/     |      \

/      |       \

PC1     PC2    Access Point

|

Wi-Fi Devices

Ask students to identify:

### Router

Connects different networks.

### Firewall

Controls traffic according to security rules.

### Switch

Connects wired LAN devices.

### Access Point

Provides wireless access.

### PCs

End devices/hosts.

# 13. Real-Time Example: College Network

Imagine a college with:

- 100 computers

- 10 printers

- Wi-Fi for students

- Internet connection

- Server

A possible design:

Internet

|

Router

|

Firewall

|

Core Switch

/    |     \

/     |      \

Computer   Server   Access Points

Switch            /    |    \

/  |  \          Wi-Fi Wi-Fi Wi-Fi

PCs PCs Printer

### What does each device do?

Router

Connects the college network to other networks/Internet.

Firewall

Applies security rules to network traffic.

Switch

Connects wired computers, servers, printers, and other LAN devices.

Access Point

Provides Wi-Fi.

# 14. Basic Troubleshooting by Device

## Problem: PC has no network connection

Check:

PC

↓

Network Cable

↓

Switch Port

↓

Switch

↓

Router/Gateway

Check:

ipconfig

Then:

ping <gateway-IP>

## Problem: Wi-Fi is not available

Check:

Access Point power

↓

Ethernet connection

↓

Switch port

↓

SSID availability

## Problem: Internet is not working but LAN works

Test:

ping <gateway-IP>

If gateway responds, test an external IP:

ping 8.8.8.8

Then check DNS:

nslookup google.com

This connects Day 6 with the network testing tools from Day 5.

# 15. Day 6 Task – Comparison Table

Students have to create a table like this:

| Device | Main Purpose | OSI Layer | Address/Technology | Common Ports/Interfaces | Example Use |
| --- | --- | --- | --- | --- | --- |
| Hub | Connect devices and repeat signals | Layer 1 | Electrical signals | Ethernet | Old/basic LAN |
| Switch | Connect LAN devices | Mainly Layer 2 | MAC address | Ethernet, SFP | College computer lab |
| Router | Connect different networks | Layer 3 | IP address | WAN, LAN, SFP | LAN to Internet |
| Firewall | Control network traffic | Varies by firewall | IP, ports, protocols, rules | Ethernet, management ports | Network security |
| Access Point | Provide Wi-Fi | Mainly Layer 2 | MAC, Wi-Fi | Ethernet, wireless | College Wi-Fi |

Important: The OSI layer shown is the primary/basic role. Modern devices can perform functions across multiple layers.

# 16. Student Observation Table

During the practical, students should fill this:

| Device | Device Model | Number of Ports | Port Types | Connected Cable | Main Function |
| --- | --- | --- | --- | --- | --- |
| Switch |  |  |  |  |  |
| Router |  |  |  |  |  |
| Firewall |  |  |  |  |  |
| Access Point |  |  |  |  |  |
| Hub |  |  |  |  |  |

# 17. Practical Questions for Students

Ask students to answer during the practical:

- Which device connects multiple computers in a LAN?

- Which device uses MAC addresses for basic frame forwarding?

- Which device connects different IP networks?

- Which device provides Wi-Fi?

- Which device controls traffic using security rules?

- What is a WAN port?

- What is a LAN port?

- What is an Ethernet port?

- What is an SFP port?

- What is a console port used for?

- Why is a switch preferred over a hub in modern LANs?

- In a college network, where would you normally find an access point?

- Which device normally acts as the default gateway for LAN hosts?

- What happens if the switch is powered off?

- What happens if the access point loses its network connection?

# 18. Day 6 Final Practical Challenge

Give students this network:

Internet

|

Router

|

Firewall

|

Switch

/    |     \

/     |      \

PC1    PC2     Access Point

/      \

Laptop   Mobile

Students have to:

- Identify every device.

- Identify the purpose of every device.

- Identify the available ports.

- Identify which devices use cables.

- Identify which devices use Wi-Fi.

- Identify where the Internet enters the network.

- Identify the default gateway device.

- Explain what happens if the switch is disconnected.

- Explain what happens if the access point is disconnected.

- Create the device comparison table.

## Day 6 Learning Outcome

After this session, students should be able to:

- Identify common network devices.

- Explain the basic function of a hub.

- Explain how a switch forwards traffic.

- Explain the purpose of a router.

- Explain the role of a firewall.

- Explain how an access point provides Wi-Fi.

- Identify common network ports and interfaces.

- Understand a basic college/office network setup.

- Select the appropriate device for a basic networking requirement.

- Connect device identification with practical troubleshooting.