Bus Topology:

# Cisco Packet Tracer – Bus Topology

## 1. Open Cisco Packet Tracer

- Open Cisco Packet Tracer.

- Select the Logical workspace.

- You will see a blank workspace.

## 2. Add Four PCs

- At the bottom-left, click End Devices.

- Select PC.

- Add 4 PCs to the workspace.

- Arrange them in a straight line.

Example:

PC1          PC2          PC3          PC4

|            |            |            |

+------------+------------+------------+

You can rename the PCs as PC1, PC2, PC3, and PC4.

## 3. Create the Bus Line

A bus topology has one main cable. All computers are connected to the same main cable.

[Terminator]

|

+---------------------------------------------+

|          |          |          |

PC1        PC2        PC3        PC4

+---------------------------------------------+

|

[Terminator]

There should be a terminator at both ends of the bus.

## 4. Connect the PCs

If your Packet Tracer version provides a Bus/Coaxial cable:

- Click Connections at the bottom-left.

- Select the Bus/Coaxial cable.

- Create the main horizontal cable.

- Connect PC1 to the main cable.

- Connect PC2 to the main cable.

- Connect PC3 to the main cable.

- Connect PC4 to the main cable.

- Add a terminator at both ends.

## 5. Set IP Addresses

All PCs should be in the same network.

### PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address:    192.168.1.1

Subnet Mask:   255.255.255.0

### PC2

IP Address:    192.168.1.2

Subnet Mask:   255.255.255.0

### PC3

IP Address:    192.168.1.3

Subnet Mask:   255.255.255.0

### PC4

IP Address:    192.168.1.4

Subnet Mask:   255.255.255.0

For this simple practical, you do not need to enter a Default Gateway.

# 6. Check Connectivity Using Ping

## Step 1 – Open PC1 Command Prompt

- Click PC1.

- Go to Desktop.

- Click Command Prompt.

## Step 2 – Ping PC2

Type:

ping 192.168.1.2

Press Enter.

If the connection is working, you should see:

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

This means PC1 can communicate with PC2.

## Step 3 – Ping PC3

From PC1, type:

ping 192.168.1.3

If you receive replies, PC1 can communicate with PC3.

## Step 4 – Ping PC4

From PC1, type:

ping 192.168.1.4

If you receive replies, PC1 can communicate with PC4.

# 7. Test from Other PCs

### From PC2

ping 192.168.1.1

ping 192.168.1.3

### From PC3

ping 192.168.1.1

ping 192.168.1.4

### From PC4

ping 192.168.1.1

ping 192.168.1.2

If you get Reply from..., the connectivity is successful.

# 8. If Ping Fails

Check these points:

- Check the IP addresses.

- Check the Subnet Mask.

- Check whether the cables are connected properly.

- Check the bus cable.

- Check the terminators.

- Make sure all PCs are in the same network.

## Final Bus Topology

[Terminator]

|

+------------------------------------------------+

|              |              |              |

PC1            PC2            PC3            PC4

|              |              |              |

+------------------------------------------------+

|

[Terminator]

## Expected Result

All PCs should be able to communicate with each other using the ping command.

Example:

PC1 → ping 192.168.1.2

PC1 → ping 192.168.1.3

PC1 → ping 192.168.1.4

Expected result:

Reply from 192.168.1.x

This confirms that the network connectivity is working.

Important: In some Cisco Packet Tracer versions, a traditional Bus/Coaxial topology is not directly supported. If you send me a screenshot of your Packet Tracer Connections menu, I can tell you exactly which cable/icon to select in your version.

Star Topology:

Sure. Here is the same practical for Star Topology in simple English, with the diagram and screenshot-style guide.

# Cisco Packet Tracer – Star Topology

## 1. Open Cisco Packet Tracer

- Open Cisco Packet Tracer.

- Select the Logical workspace.

- You will see a blank workspace.

## 2. Add One Switch

At the bottom-left:

Network Devices → Switches

Select:

2960 Switch

Place it in the center of the workspace.

## 3. Add Four PCs

Go to:

End Devices → PC

Add 4 PCs.

Arrange them around the switch:

PC1

|

|

PC2 ------------- Switch ------------- PC3

|

|

PC4

This is called a Star Topology because all devices are connected to one central device.

## 4. Connect the PCs to the Switch

- Click Connections at the bottom-left.

- Select Copper Straight-Through cable.

- Click PC1.

- Select FastEthernet0.

- Click the Switch.

- Select FastEthernet0/1.

Now connect the other PCs:

PC1 → Switch Fa0/1

PC2 → Switch Fa0/2

PC3 → Switch Fa0/3

PC4 → Switch Fa0/4

Final diagram:

PC1

|

|

|

PC2 ---------------- SWITCH ---------------- PC3

|

|

|

PC4

## 5. Set IP Addresses

All PCs should be in the same network.

### PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address:    192.168.1.1

Subnet Mask:   255.255.255.0

### PC2

IP Address:    192.168.1.2

Subnet Mask:   255.255.255.0

### PC3

IP Address:    192.168.1.3

Subnet Mask:   255.255.255.0

### PC4

IP Address:    192.168.1.4

Subnet Mask:   255.255.255.0

You do not need to enter a Default Gateway for this simple network.

# 6. Check Connectivity Using Ping

## From PC1 to PC2

- Click PC1.

- Go to Desktop.

- Click Command Prompt.

- Type:

ping 192.168.1.2

Press Enter.

You should get:

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

This means PC1 and PC2 are connected successfully.

## From PC1 to PC3

ping 192.168.1.3

## From PC1 to PC4

ping 192.168.1.4

If you get Reply from..., the connection is working.

# 7. Test from Other PCs

### From PC2

ping 192.168.1.1

ping 192.168.1.3

### From PC3

ping 192.168.1.1

ping 192.168.1.4

### From PC4

ping 192.168.1.1

ping 192.168.1.2

# 8. Expected Result

All four PCs should be able to communicate with each other through the central switch.

PC1

|

|

|

PC2 ------------- SWITCH ------------- PC3

|

|

|

PC4

If all ping commands show Reply from..., your Star Topology is working successfully.

# 9. If Ping Fails

Check the following:

- Check the IP addresses.

- Check the subnet mask.

- Make sure you used Copper Straight-Through cable.

- Check that all cables are connected to the correct switch ports.

- Make sure the switch is connected to all four PCs.

- Make sure all PCs are in the same network.

## Port Connection Table

| Device | Switch Port |
| --- | --- |
| PC1 | Fa0/1 |
| PC2 | Fa0/2 |
| PC3 | Fa0/3 |
| PC4 | Fa0/4 |

## IP Address Table

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.1.1 | 255.255.255.0 |
| PC2 | 192.168.1.2 | 255.255.255.0 |
| PC3 | 192.168.1.3 | 255.255.255.0 |
| PC4 | 192.168.1.4 | 255.255.255.0 |

Screenshot guide: The generated screenshot-style diagram above shows the exact flow: Add Switch + 4 PCs → Connect with Copper Straight-Through → Set IPs → Open Command Prompt → Ping → Check Reply.

Ring Topology

# Cisco Packet Tracer – Ring Topology

## 1. Open Cisco Packet Tracer

- Open Cisco Packet Tracer.

- Select the Logical workspace.

- You will see a blank workspace.

# 2. Add Four Switches

Go to:

Network Devices → Switches

Select:

2960 Switch

Add 4 switches to the workspace.

Rename them:

S1

S2

S3

S4

Arrange them in a square or ring shape:

S1

/    \

/      \

S4        S2

\      /

\    /

S3

# 3. Add Four PCs

Go to:

End Devices → PC

Add 4 PCs.

Connect one PC to each switch.

PC1

|

S1

/    \

/      \

S4        S2

|          |

PC4        PC2

\      /

\    /

S3

|

PC3

# 4. Connect PCs to Switches

Click:

Connections → Copper Straight-Through

Connect:

PC1 → S1

PC2 → S2

PC3 → S3

PC4 → S4

Use the following switch ports:

PC1 → S1 Fa0/1

PC2 → S2 Fa0/1

PC3 → S3 Fa0/1

PC4 → S4 Fa0/1

# 5. Connect the Switches in a Ring

Now connect the switches with Copper Cross-Over cable.

Connect:

S1 → S2

S2 → S3

S3 → S4

S4 → S1

Use these ports:

S1 Fa0/2 → S2 Fa0/2

S2 Fa0/3 → S3 Fa0/2

S3 Fa0/3 → S4 Fa0/2

S4 Fa0/3 → S1 Fa0/3

The final ring should look like:

S1

/    \

/      \

S4        S2

\      /

\    /

S3

The important point is:

Every switch must be connected to two other switches.

# 6. Set IP Addresses

All PCs should be in the same network.

## PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address:     192.168.1.1

Subnet Mask:    255.255.255.0

## PC2

IP Address:     192.168.1.2

Subnet Mask:    255.255.255.0

## PC3

IP Address:     192.168.1.3

Subnet Mask:    255.255.255.0

## PC4

IP Address:     192.168.1.4

Subnet Mask:    255.255.255.0

For this simple network, Default Gateway is not required.

# 7. Wait for the Connections to Become Green

After connecting the devices, wait for a few seconds.

The connection indicators should become green.

If the links are orange for a short time, wait.

If they remain red, check the cable and port connections.

# 8. Check Connectivity Using Ping

## Test PC1 → PC2

- Click PC1.

- Go to Desktop.

- Open Command Prompt.

- Type:

ping 192.168.1.2

Press Enter.

You should get:

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

Reply from 192.168.1.2

This means PC1 can communicate with PC2.

# 9. Test PC1 → PC3

From PC1 Command Prompt:

ping 192.168.1.3

You should get:

Reply from 192.168.1.3

# 10. Test PC1 → PC4

Type:

ping 192.168.1.4

You should get:

Reply from 192.168.1.4

# 11. Test from Other PCs

## From PC2

ping 192.168.1.1

ping 192.168.1.3

## From PC3

ping 192.168.1.1

ping 192.168.1.4

## From PC4

ping 192.168.1.1

ping 192.168.1.2

If you receive Reply from..., the connectivity is working.

# 12. Final Ring Topology

Your final topology should look similar to this:

PC1

|

S1

/    \

/      \

S4        S2

|          |

PC4        PC2

\      /

\    /

S3

|

PC3

The four switches form a closed ring:

S1 → S2 → S3 → S4 → S1

# 13. Device Connection Table

| Device | Connected To |
| --- | --- |
| PC1 | S1 |
| PC2 | S2 |
| PC3 | S3 |
| PC4 | S4 |
| S1 | S2 and S4 |
| S2 | S1 and S3 |
| S3 | S2 and S4 |
| S4 | S3 and S1 |

# 14. IP Address Table

| PC | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.1.1 | 255.255.255.0 |
| PC2 | 192.168.1.2 | 255.255.255.0 |
| PC3 | 192.168.1.3 | 255.255.255.0 |
| PC4 | 192.168.1.4 | 255.255.255.0 |

# 15. If Ping Fails

Check these points:

- Check the IP addresses.

- Check the subnet mask.

- Check the cables.

- Check the switch ports.

- Make sure every switch is connected to two other switches.

- Make sure the ring is closed.

- Wait for the links to become green.

# Expected Result

All four PCs should be able to communicate with each other.

For example:

PC1 → PC2     Successful

PC1 → PC3     Successful

PC1 → PC4     Successful

PC2 → PC3     Successful

PC2 → PC4     Successful

PC3 → PC4     Successful

A successful ping shows:

Reply from 192.168.1.x

This confirms that the Ring Topology is working successfully.

## Mesh Topology – Cisco Packet Tracer Steps

### Step 1: Add PCs

- Open Cisco Packet Tracer.

- Go to End Devices.

- Select PC.

- Drag and drop 4 PCs into the workspace.

- Rename them:

  - PC1

  - PC2

  - PC3

  - PC4

### Step 2: Add Extra Ports

- Click PC1.

- Open Physical tab.

- Turn Power OFF.

- Add network modules to get extra Ethernet ports.

- Turn Power ON.

- Repeat the same steps for PC2, PC3, and PC4.

### Step 3: Connect PC1 to PC2

- Click Connections (lightning icon).

- Select Copper Cross-Over.

- Click PC1 → select an available FastEthernet port.

- Click PC2 → select an available FastEthernet port.

### Step 4: Connect PC1 to PC3

PC1 → PC3

Use another available port on both PCs.

### Step 5: Connect PC1 to PC4

PC1 → PC4

Use another available port.

### Step 6: Connect PC2 to PC3

PC2 → PC3

### Step 7: Connect PC2 to PC4

PC2 → PC4

### Step 8: Connect PC3 to PC4

PC3 → PC4

### Final Connections

PC1 ───── PC2

│ \       /│

│  \     / │

│   \   /  │

│    \ /   │

PC3 ───── PC4

There should be 6 direct connections in total.

### Step 9: Test the Topology

- Click PC1.

- Go to Desktop → Command Prompt.

- Use ping with the IP address of another PC.

- Repeat the test from other PCs.

- If you receive Reply from..., the connection is working.