# Day 5: Troubleshooting Network Services

Duration: 1 Hour | Theory: 15 Minutes | Practical: 45 Minutes

Topic: Troubleshooting DNS and DHCP Issues in a Simulated Network

Objective: Students will learn how to identify, troubleshoot, and resolve common DNS and DHCP problems using Cisco Packet Tracer and Windows Command Prompt.

## 1. Introduction to Network Troubleshooting

Network troubleshooting is the process of identifying and fixing problems that prevent devices from communicating properly over a network.

Real-time example:

Imagine an office has 20 computers. Suddenly:

- Some computers cannot access websites.

- Some computers are not receiving IP addresses automatically.

- Users can access a website using its IP address but not using its domain name.

A network administrator must identify the actual problem and resolve it.

Basic troubleshooting process:

## 2. Common DNS Issues

DNS stands for Domain Name System. It converts domain names into IP addresses.

For example:

www.google.com → IP Address

When DNS does not work, users may be unable to access websites using domain names.

### Common DNS problems and solutions

| Problem | Possible reason | Solution |
| --- | --- | --- |
| Website not opening by domain name | Incorrect DNS address | Configure the correct DNS server IP |
| Website opens using IP but not domain | DNS resolution failure | Check DNS service and A record |
| DNS server not responding | DNS service is OFF | Turn DNS service ON |
| Incorrect website IP returned | Wrong DNS record | Correct the A record |
| Domain name not resolving | Incorrect domain name | Verify the domain spelling |
| Old IP address returned | Cached DNS record | Clear DNS cache and retry |

### Important DNS troubleshooting commands

Windows CMD:

ipconfig /all

Displays IP address, DNS server, gateway and other network configuration.

nslookup google.com

Checks whether a domain name resolves to an IP address.

ipconfig /flushdns

Clears the local DNS cache.

ping 192.168.10.2

Checks connectivity to the DNS server.

Important: A successful ping does not always mean DNS is working. DNS resolution must be tested separately.

## 3. Common DHCP Issues

DHCP stands for Dynamic Host Configuration Protocol.

DHCP automatically provides network configuration to client devices, including:

- IP address

- Subnet mask

- Default gateway

- DNS server address

### Common DHCP problems and solutions

| Problem | Possible reason | Solution |
| --- | --- | --- |
| Computer gets 169.254.x.x address | DHCP server not responding | Check DHCP server and network connectivity |
| Computer cannot obtain IP | DHCP service is OFF | Enable DHCP service |
| Duplicate IP address | Incorrect DHCP pool or static IP conflict | Correct the IP allocation range |
| Incorrect gateway | Wrong DHCP pool configuration | Set the correct default gateway |
| Incorrect DNS address | Wrong DNS option in DHCP pool | Update DNS server address |
| DHCP server not reachable | Network or VLAN issue | Check cables, switch ports and VLAN |

Important: A Windows computer receiving an address beginning with 169.254 usually means it could not obtain an IPv4 address from a DHCP server. This is called APIPA.

### Important DHCP troubleshooting commands

ipconfig /all

Displays the current DHCP and IP configuration.

ipconfig /release

Releases the current DHCP-assigned IPv4 address.

ipconfig /renew

Requests a new IP address from the DHCP server.

ipconfig /all

Run again to verify the new configuration.

# PART A: PRACTICAL (45 MINUTES)

## Practical: Troubleshoot DNS and DHCP Issues in Cisco Packet Tracer

Hands-on lab

Objective: Create a network with a DHCP and DNS server, intentionally introduce network configuration errors, identify the problems, and resolve them.

## 4. Network Topology

Required devices:

- 1 Server-PT

- 1 Switch 2960

- 3 PCs

- Copper Straight-Through cables

### IP Address Plan

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| Server | 192.168.10.2 | 255.255.255.0 |
| PC0 | DHCP | Automatic |
| PC1 | DHCP | Automatic |
| PC2 | DHCP | Automatic |

DNS Server IP: 192.168.10.2

Default Gateway: 192.168.10.1 (for this lab, no router is required for same-LAN testing; the gateway is only a configured option).

## 5. Step 1: Configure the Server

Click:

Server → Desktop → IP Configuration

Enter:

IP Address:      192.168.10.2

Subnet Mask:     255.255.255.0

Default Gateway: 192.168.10.1

DNS Server:      192.168.10.2

## 6. Step 2: Configure DHCP Service

Click:

Server → Services → DHCP

Turn DHCP Service ON.

Create the following pool:

| Field | Value |
| --- | --- |
| Pool Name | STUDENT_LAB |
| Default Gateway | 192.168.10.1 |
| DNS Server | 192.168.10.2 |
| Start IP Address | 192.168.10.100 |
| Subnet Mask | 255.255.255.0 |
| Maximum Users | 50 |

Click Add or Save, depending on the Packet Tracer version.

## 7. Step 3: Configure DNS Service

Click:

Server → Services → DNS

- Turn DNS Service ON.

- Add the following DNS record.

| Field | Value |
| --- | --- |
| Name | www.studentlab.com |
| Type | A Record |
| Address | 192.168.10.2 |

Click Add.

Also enable the HTTP service on the same server:

Server → Services → HTTP → ON

This allows students to test the domain name using the web browser.

## 8. Step 4: Configure PCs Using DHCP

For each PC:

- Click PC0.

- Open Desktop.

- Select IP Configuration.

- Click DHCP.

Repeat for PC1 and PC2.

Expected configuration:

| PC | Expected IP |
| --- | --- |
| PC0 | 192.168.10.100 |
| PC1 | 192.168.10.101 |
| PC2 | 192.168.10.102 |

The actual addresses may differ depending on the order in which devices request leases.

## 9. Step 5: Test the Network

On PC0, open Command Prompt.

Check IP configuration:

ipconfig

Ping the server:

ping 192.168.10.2

Test domain resolution:

ping www.studentlab.com

Open PC0 → Desktop → Web Browser.

Enter:

http://www.studentlab.com

If everything is configured correctly, the server's web page should open.

# PART B: TROUBLESHOOTING CHALLENGES

Now students must intentionally create problems and solve them.

## Challenge 1: DNS Server Not Working

Problem: The user can ping the server using its IP address but cannot access the website using its domain name.

Trainer creates the issue:

- Open Server → Services → DNS.

- Turn DNS Service OFF.

Student task:

- Test ping 192.168.10.2.

- Test ping www.studentlab.com.

- Identify why the domain name is not resolving.

- Enable DNS Service.

- Test the domain again.

Expected solution: DNS Service must be ON and the correct A record must exist.

## Challenge 2: Incorrect DNS Record

Problem: The website domain resolves to the wrong IP address.

Trainer creates the issue:

Change the DNS A record:

www.studentlab.com → 192.168.10.50

Actual server IP:

192.168.10.2

Student task:

- Test the domain name.

- Identify the incorrect IP address.

- Correct the DNS record.

- Test again.

Expected solution:

www.studentlab.com → 192.168.10.2

## Challenge 3: DHCP Service Disabled

Problem: New computers are not receiving IP addresses automatically.

Trainer creates the issue:

- Open Server → Services → DHCP.

- Turn DHCP Service OFF.

Student task:

- Open PC0 → Desktop → IP Configuration.

- Select DHCP.

- Observe the result.

- Identify why the PC cannot obtain an IP address.

- Enable DHCP Service.

- Request an IP address again.

Expected solution: Enable DHCP Service and renew the client configuration.

## Challenge 4: Incorrect DHCP DNS Configuration

Problem: PCs receive IP addresses successfully, but domain names do not resolve.

Trainer creates the issue:

In the DHCP pool, change the DNS Server value to:

192.168.10.50

Actual DNS server:

192.168.10.2

Student task:

- Check the PC's DNS address.

- Identify the incorrect DNS configuration.

- Correct the DHCP pool.

- Renew the PC's DHCP configuration.

- Test the domain name again.

Expected solution: Configure the correct DNS server IP in the DHCP pool and renew the client lease.

## Challenge 5: Incorrect DHCP Gateway

Problem: A PC receives an IP address but has an incorrect default gateway.

Trainer creates the issue:

Change the DHCP pool gateway to:

192.168.10.254

Correct gateway:

192.168.10.1

Student task:

- Check the default gateway using ipconfig.

- Identify the incorrect gateway.

- Correct the DHCP pool.

- Renew the client configuration.

- Verify the new gateway.

Expected solution: Update the default gateway in the DHCP pool.

Trainer's note

For challenges 4 and 5, students should release and renew their DHCP configuration after correcting the pool. Existing leases may need to be renewed before the new DHCP options appear.

ipconfig /release

ipconfig /renew

# PART C: STUDENT TASK – TROUBLESHOOTING REPORT

## 10. Document Solutions for Common DNS and DHCP Issues

Each student must prepare a troubleshooting report based on the practical exercises.

Use this format:

| No. | Problem | Cause | Solution | Verification |
| --- | --- | --- | --- | --- |
| 1 | DNS not resolving | DNS service OFF | Enable DNS | Domain resolves |
| 2 | Wrong IP returned | Incorrect A record | Correct A record | Correct IP displayed |
| 3 | DHCP not assigning IP | DHCP service OFF | Enable DHCP | Client receives IP |
| 4 | Wrong DNS on client | Incorrect DHCP DNS option | Correct DNS IP | Domain resolves |
| 5 | Wrong gateway | Incorrect DHCP gateway | Correct gateway | Gateway verified |

### Student Assignment Questions

- What is network troubleshooting?

- What is the difference between DNS and DHCP?

- What happens when the DNS server is not responding?

- Why does a computer receive a 169.254.x.x address?

- What is the purpose of ipconfig /release?

- What is the purpose of ipconfig /renew?

- How can you identify an incorrect DNS server address?

- What happens if the DHCP gateway is incorrect?

- Why can a user access a server using an IP address but not its domain name?

- Explain the troubleshooting process with one real-time example.

### Submission Requirements

- Packet Tracer network topology screenshot.

- DHCP configuration screenshot.

- DNS configuration screenshot.

- Successful IP configuration screenshot.

- Screenshots of troubleshooting before and after fixing issues.

- Completed DNS and DHCP troubleshooting report.

- Save Packet Tracer file as Day5_DNS_DHCP_Troubleshooting.pkt.

## 11. Quick Revision

| Command / Concept | Purpose |
| --- | --- |
| ipconfig /all | View complete network configuration |
| ipconfig /release | Release DHCP IP address |
| ipconfig /renew | Request a new DHCP IP address |
| ipconfig /flushdns | Clear DNS cache |
| nslookup domain.com | Test DNS resolution |
| ping IP | Test network connectivity |
| DNS | Converts domain names into IP addresses |
| DHCP | Automatically assigns IP configuration |
| APIPA | Automatic IPv4 address when DHCP assignment fails |

End-of-day discussion:

Ask students:

"If you are working as a network administrator and a user says that the internet is not working, what will you check first: IP address, DNS, DHCP, or gateway? Explain your troubleshooting steps."

This helps students think like real network support engineers instead of simply memorizing commands.