# Day 1: Introduction to Network Protocols

Duration: 2 Hours
Theory: 1 Hour
Practical: 1 Hour

## 1. Learning Objectives

By the end of this session, students will be able to:

- Understand the meaning and importance of network protocols.

- Explain DNS, DHCP, HTTP, HTTPS, FTP, and SNMP.

- Understand how DNS and HTTP work during website access.

- Capture and observe DNS and HTTP traffic using Wireshark.

- Prepare a report on DNS and HTTP communication.

# Part A: Theory (1 Hour)

## 2. What is a Network Protocol?

A network protocol is a set of rules that allows two or more devices to communicate with each other over a network.

When we open a website, send an email, or transfer a file, different network protocols work in the background.

Real-time example:

When you open www.google.com in a browser:

- DNS finds the IP address of the website.

- TCP helps establish a reliable connection when TCP is used.

- HTTPS provides secure communication between your browser and the website.

- HTTP defines how web requests and responses are exchanged.

Without network protocols, devices would not have a common set of rules for exchanging data.

## 3. Types of Network Protocols

### A. DNS (Domain Name System)

DNS converts a human-readable domain name into an IP address.

For example, humans can easily remember www.google.com, but computers use IP addresses to communicate with servers.

User enters a website

www.example.com

DNS Lookup

Finds the website's IP address

Web Server IP Address

The browser can now contact the server

Real-time example: When you type www.amazon.com into a browser, DNS helps find the IP address associated with that domain.

Important points:

- DNS stands for Domain Name System.

- DNS commonly uses port 53.

- It usually uses UDP for queries, but TCP is also used in certain situations.

- DNS records include A, AAAA, CNAME, and MX records.

### B. DHCP (Dynamic Host Configuration Protocol)

DHCP automatically provides network configuration to devices when they join a network.

This configuration can include:

- IP address

- Subnet mask

- Default gateway

- DNS server address

Real-time example: When your mobile phone connects to your home Wi-Fi, the router's DHCP server can automatically assign an IP address to your phone.

Important points:

- DHCP uses UDP.

- Server port: 67

- Client port: 68

- It reduces the need to configure IP addresses manually.

The basic DHCP process is called DORA:

## D

Discover

The device searches for a DHCP server.

## O

Offer

The server offers an IP address.

## R

Request

The device requests the offered address.

## A

Acknowledge

The server confirms the IP assignment.

### C. HTTP (Hypertext Transfer Protocol)

HTTP is used for communication between web browsers and web servers.

It defines how the browser sends requests and how the server returns responses.

Real-time example: When you open a news website, your browser sends an HTTP request and the server returns the requested webpage or other content.

Common HTTP methods:

- GET: Retrieve data.

- POST: Submit data.

- PUT: Create or update a resource, depending on the API.

- DELETE: Delete a resource.

HTTP commonly uses port 80.

Example of web communication:

Browser                  Web Server

|                          |

|------ HTTP Request ------>|

|                          |

|<----- HTTP Response -----|

|                          |

Display webpage to user

### D. HTTPS (Hypertext Transfer Protocol Secure)

HTTPS is the secure version of HTTP. It uses TLS to protect communication between a browser and a server.

It provides:

- Confidentiality: Helps prevent others from reading the transmitted data.

- Integrity: Helps detect unauthorized changes to data.

- Authentication: Helps verify the server's identity through its certificate.

Real-time example: When you log in to an online banking website, HTTPS helps protect your username, password, and other transmitted information.

Important points:

- HTTPS commonly uses port 443.

- It uses TLS for secure communication.

- HTTPS encrypts HTTP messages in transit, unlike ordinary HTTP.

| HTTP | HTTPS |
| --- | --- |
| Commonly uses port 80 | Commonly uses port 443 |
| No TLS protection by default | Uses TLS protection |
| Data is not encrypted by HTTP itself | HTTP messages are protected in transit |

### E. FTP (File Transfer Protocol)

FTP is a protocol used to transfer files between a client and a server.

Real-time example: A developer uploads website files from a computer to a hosting server using an FTP client.

Common uses:

- Uploading website files.

- Downloading files from a server.

- Managing files on a remote system.

Important points:

- FTP traditionally uses TCP.

- Port 21 is used for the control connection.

- Traditional active FTP commonly uses port 20 for data, while passive FTP uses negotiated data ports.

- Standard FTP does not encrypt credentials or data. Secure alternatives include SFTP and FTPS.

### F. SNMP (Simple Network Management Protocol)

SNMP is used to monitor and manage network devices.

Network administrators can use SNMP to collect information about routers, switches, servers, printers, and other supported devices.

Real-time example: A network administrator monitors switch status, interface traffic, and device health from a central monitoring dashboard.

Common uses:

- Monitoring network devices.

- Checking interface status.

- Collecting bandwidth statistics.

- Receiving alerts about network problems.

Important points:

- SNMP commonly uses UDP.

- Port 161 is generally used for queries and responses.

- Port 162 is generally used for notifications or traps.

- SNMPv3 supports authentication and encryption when properly configured.

## 4. Quick Revision Table

| Protocol | Main purpose | Common port |
| --- | --- | --- |
| DNS | Resolves domain names to IP addresses | 53 |
| DHCP | Automatically assigns IP configuration | UDP 67/68 |
| HTTP | Web communication | 80 |
| HTTPS | Secure web communication | 443 |
| FTP | File transfer | TCP 21 control |
| SNMP | Network monitoring and management | UDP 161/162 |

Note: These are the common/default ports. Actual network configurations can vary.

## 5. How DNS and HTTP Work Together

Consider a student opening http://example.com in a browser.

Step 1: Browser

Student enters http://example.com

Step 2: DNS

Resolves example.com to an IP address if needed

Step 3: Web Connection

The browser connects to the server

Step 4: HTTP Request

The browser requests a webpage

Step 5: HTTP Response

The server returns webpage content

The important difference is that DNS helps the browser find the server, while HTTP defines the exchange of webpage requests and responses.

In practice, browsers may use cached DNS results, and websites may use HTTPS instead of HTTP.

# Part B: Practical (1 Hour)

## Practical: Observe DNS and HTTP Traffic Using Wireshark

Objective: Capture network packets and identify DNS queries, DNS responses, HTTP requests, and HTTP responses.

Requirements:

- Windows computer

- Wireshark installed

- Internet connection

- Web browser

### Step 1: Open Wireshark

- Open Wireshark.

- Identify the active network interface, such as Wi-Fi or Ethernet.

- Double-click the active interface to start capturing packets.

### Step 2: Generate DNS Traffic

Open Command Prompt and execute:

nslookup example.com

Observe the output. It should show the DNS server used and the resolved address or addresses, if resolution succeeds.

Return to Wireshark and enter this display filter:

dns

Look for DNS query and response packets.

Check the following details:

- Source IP address

- Destination IP address

- Queried domain name

- DNS query type, such as A or AAAA

- Resolved IP address in the response, if present

Expected observation: Wireshark may display a DNS query for example.com and a response containing one or more IP addresses.

If no new query appears, the result may be cached. Try another domain or use nslookup with a different hostname.

### Step 3: Generate HTTP Traffic

For this exercise, use a test website that supports plain HTTP, such as:

http://neverssl.com

Open the address in your browser while Wireshark is capturing packets.

Apply this display filter:

http

Look for HTTP packets, if the browser and network actually generate plain HTTP traffic.

Check the following details:

- HTTP request method, such as GET

- Requested hostname or URI

- HTTP response status code, if a response is captured

- Source and destination IP addresses

For example, an HTTP request may contain:

GET / HTTP/1.1

Host: neverssl.com

The server may return a response such as HTTP/1.1 200 OK, although the actual response can vary.

Important: Do not use an HTTPS-only website for this part. HTTPS encrypts the HTTP content, so ordinary Wireshark captures generally cannot display the readable HTTP request and response headers. You may still observe connection metadata.

### Step 4: Compare DNS and HTTP Packets

| Observation | DNS | HTTP |
| --- | --- | --- |
| Purpose | Find the IP address for a domain | Request and receive web content |
| Typical port | 53 | 80 |
| What to look for | Query name and response address | Method, URI, status code |
| Common use | Before connecting to a server, if resolution is needed | When exchanging webpage content |

### Step 5: Stop and Save the Capture

- Click the red Stop button in Wireshark.

- Go to File → Save As.

- Save the capture as Day1_DNS_HTTP.pcapng.

Students should keep the capture file for later analysis.