# Inter-VLAN Routing Practical in Cisco Packet Tracer

## 1. Practical Objective

To configure Inter-VLAN Routing in Cisco Packet Tracer so that computers connected to different VLANs can communicate with each other through a router.

In this practical, we use the Router-on-a-Stick method.

## 2. Devices Required

- 1 Router

- 1 Switch

- 2 PCs

- Copper Straight-Through cables

## 3. Network Topology and Connections

Connect the devices as follows:

| Device 1 | Port | Device 2 | Port |
| --- | --- | --- | --- |
| Router | FastEthernet0/0 | Switch | FastEthernet0/1 |
| PC0 | FastEthernet0 | Switch | FastEthernet1/1 |
| PC1 | FastEthernet0 | Switch | FastEthernet2/1 |

Important: In this practical, the switch has ports such as Fa1/1 and Fa2/1. Always use the actual port names displayed on your switch.

## 4. IP Addressing Table

| Device | VLAN | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| PC0 | VLAN 10 | 192.168.10.10 | 255.255.255.0 | 192.168.10.1 |
| PC1 | VLAN 20 | 192.168.20.10 | 255.255.255.0 | 192.168.20.1 |
| Router Subinterface Fa0/0.10 | VLAN 10 | 192.168.10.1 | 255.255.255.0 | Not required |
| Router Subinterface Fa0/0.20 | VLAN 20 | 192.168.20.1 | 255.255.255.0 | Not required |

## 5. Step 1: Configure VLANs on the Switch

Open the switch CLI and enter the following commands:

enable

configure terminal

vlan 10

name SALES

exit

vlan 20

name HR

exit

VLAN 10 is created for the SALES department, and VLAN 20 is created for the HR department.

## 6. Step 2: Assign Switch Ports to VLANs

Assign PC0's port to VLAN 10:

interface fastEthernet 1/1

switchport mode access

switchport access vlan 10

no shutdown

exit

Assign PC1's port to VLAN 20:

interface fastEthernet 2/1

switchport mode access

switchport access vlan 20

no shutdown

exit

Configure the switch port connected to the router as a trunk:

interface fastEthernet 0/1

switchport mode trunk

no shutdown

exit

end

The trunk port carries traffic for multiple VLANs between the switch and the router.

## 7. Step 3: Configure the Router

Open the router CLI and enter the following commands.

First, enable the physical interface:

enable

configure terminal

interface fastEthernet 0/0

no shutdown

exit

Create the subinterface for VLAN 10:

interface fastEthernet 0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

exit

Create the subinterface for VLAN 20:

interface fastEthernet 0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

exit

end

The router uses these subinterfaces as gateways for the two VLANs.

## 8. Step 4: Configure PC IP Addresses

### PC0 Configuration

Open PC0 → Desktop → IP Configuration.

Enter:

- IP Address: 192.168.10.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.10.1

### PC1 Configuration

Open PC1 → Desktop → IP Configuration.

Enter:

- IP Address: 192.168.20.10

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.20.1

Make sure both PCs have the correct IP address, subnet mask, and default gateway.

## 9. Step 5: Verify Switch Configuration

Run the following commands on the switch.

### Check VLAN Assignment

show vlan brief

Expected important entries:

- VLAN 10 — SALES — Fa1/1

- VLAN 20 — HR — Fa2/1

### Check Trunk Configuration

show interfaces trunk

Expected result:

- Port: Fa0/1

- Mode: on

- Encapsulation: 802.1q

- Status: trunking

- VLANs allowed and active: VLAN 1, 10, and 20

### Check Port Status

show interfaces status

Expected important entries:

- Fa0/1 — connected — trunk

- Fa1/1 — connected — VLAN 10

- Fa2/1 — connected — VLAN 20

## 10. Step 6: Verify Router Configuration

Run the following command on the router:

show ip interface brief

Expected important entries:

| Interface | IP Address | Status | Protocol |
| --- | --- | --- | --- |
| FastEthernet0/0 | unassigned | up | up |
| FastEthernet0/0.10 | 192.168.10.1 | up | up |
| FastEthernet0/0.20 | 192.168.20.1 | up | up |

The physical interface and both subinterfaces should be up/up.

To check the routing table, run:

show ip route

The router should have connected routes for:

- 192.168.10.0/24

- 192.168.20.0/24

## 11. Step 7: Perform the Ping Test

Open PC0 → Desktop → Command Prompt.

First, test the local VLAN gateway:

ping 192.168.10.1

Then, test communication with PC1 in the other VLAN:

ping 192.168.20.10

Expected Result: The ping should receive replies from the destination IP address.

If PC0 can successfully ping PC1, Inter-VLAN Routing is working.

You can also test the reverse direction from PC1:

ping 192.168.20.1

ping 192.168.10.10

## 12. Troubleshooting Checkpoints

During the practical, a few minor issues may occur. Use these checkpoints before changing the configuration.

### Checkpoint 1: Invalid Input Detected

Problem: A command such as show ip interface brief displays:

% Invalid input detected at '^' marker.

Reason: The command was entered in configuration mode, such as Router(config)#.

Solution: Exit to privileged EXEC mode:

end

show ip interface brief

Alternatively, use do while in configuration mode:

do show ip interface brief

### Checkpoint 2: Router Interface Is Down

Problem: The router's physical interface displays down/down or administratively down/down.

Solution:

enable

configure terminal

interface fastEthernet 0/0

no shutdown

end

Also check that the cable connects Router Fa0/0 to Switch Fa0/1.

### Checkpoint 3: Trunk Is Not Working

Problem: show interfaces trunk does not display the expected trunk port.

Check:

- The router is connected to the correct switch port.

- The correct port has been configured as a trunk.

- The physical cable is connected.

- The router interface is enabled.

Use the following commands on the switch:

configure terminal

interface fastEthernet 0/1

switchport mode trunk

no shutdown

end

### Checkpoint 4: PC Ports Are in the Wrong VLAN

Problem: A PC is connected but assigned to the wrong VLAN.

Solution: Verify the VLAN assignment:

show vlan brief

For PC0:

configure terminal

interface fastEthernet 1/1

switchport mode access

switchport access vlan 10

end

For PC1:

configure terminal

interface fastEthernet 2/1

switchport mode access

switchport access vlan 20

end

### Checkpoint 5: First Ping Times Out

Problem: The first ping fails, but the configuration appears correct.

Reason: The devices may need to resolve MAC addresses using ARP.

Solution: Run the same ping command again and check the result.

If repeated pings fail, verify the IP configuration, VLAN assignment, trunk, and router subinterfaces.

### Checkpoint 6: Ping Between VLANs Fails

Check these items in order:

- PC0 IP: 192.168.10.10

- PC0 gateway: 192.168.10.1

- PC1 IP: 192.168.20.10

- PC1 gateway: 192.168.20.1

- Switch Fa1/1 belongs to VLAN 10.

- Switch Fa2/1 belongs to VLAN 20.

- Switch Fa0/1 is trunking.

- Router Fa0/0, Fa0/0.10, and Fa0/0.20 are up/up.

- The VLAN IDs in encapsulation dot1Q match the switch VLAN IDs.

Do not change a working configuration unnecessarily. Identify the failed checkpoint first.

## 13. Final Verification Checklist

VLAN 10 and VLAN 20 are created.

PC0 is assigned to VLAN 10.

PC1 is assigned to VLAN 20.

The switch port connected to the router is trunking.

Router subinterfaces have the correct IP addresses.

Router interfaces show up/up.

Both PCs have the correct default gateways.

PC0 can ping 192.168.10.1.

PC0 can ping 192.168.20.10.

## 14. Conclusion

In this practical, two separate VLANs were created and configured on a switch. The router was configured using the Router-on-a-Stick method, with one subinterface for each VLAN. After assigning the correct IP addresses and default gateways, the ping test succeeded between PC0 and PC1.

Therefore, Inter-VLAN Routing was configured successfully in Cisco Packet Tracer.