Day 4: Network Cable Types and Tools

Duration: 1 Hour

Theory Session (30 Minutes)

Topic: Types of Cables (Ethernet, Fiber Optics) and Networking Tools

Learning Objectives

After completing this session, students will be able to:

• Identify different types of network cables.
• Understand the purpose of Ethernet and Fiber Optic cables.
• Recognize networking tools used for cable installation.
• Understand RJ45 connectors and crimping tools.
• Prepare a straight-through Ethernet cable.

Introduction

Network cables are used to connect computers and networking devices such as routers, switches, and access points. They provide a physical path for data transmission.

Types of Network Cables

- Ethernet Cable (UTP Cable)

Ethernet cable is the most commonly used network cable in homes and offices.

Features

• Uses copper wires.
• Easy to install.
• Cost-effective.
• Supports high-speed communication.

Common Categories

Cat5e
• Speed up to 1 Gbps

Cat6
• Speed up to 10 Gbps (short distances)

Cat6a
• Improved performance and reduced interference

Uses

• Computer to Switch
• Computer to Router
• Switch to Router
• Access Point Connections

RJ45 Connector

The RJ45 connector is attached to both ends of an Ethernet cable and plugs into network ports.

Features

• 8 Pins
• Standard connector for Ethernet networks
• Supports data communication

- Fiber Optic Cable

Fiber optic cable uses light signals instead of electrical signals.

Features

• Very high speed
• Long-distance communication
• Resistant to electrical interference

Uses

• Internet Service Providers (ISPs)
• Data Centers
• Enterprise Networks
• Long-distance communication links

Advantages of Fiber Optics

• Faster data transmission
• Greater bandwidth
• Better reliability
• Longer cable lengths

Comparison of Ethernet and Fiber Optic Cables

Ethernet Cable

• Uses copper wires
• Lower cost
• Easier installation
• Suitable for homes and offices

Fiber Optic Cable

• Uses light signals
• Higher cost
• Higher speed
• Suitable for large networks

Networking Tools

- RJ45 Crimping Tool

Purpose

Used to attach RJ45 connectors to Ethernet cables.

Functions

• Cutting cable
• Stripping cable insulation
• Crimping RJ45 connectors

- Cable Stripper

Purpose

Removes the outer insulation from network cables without damaging internal wires.

- LAN Cable Tester

Purpose

Tests whether an Ethernet cable is wired correctly.

Can identify:

• Open circuits
• Short circuits
• Incorrect wiring

- RJ45 Connectors

Purpose

Used to terminate Ethernet cables and connect them to networking devices.

Ethernet Cable Wiring Standards

T568A Standard

- White/Green

- Green

- White/Orange

- Blue

- White/Blue

- Orange

- White/Brown

- Brown

T568B Standard

- White/Orange

- Orange

- White/Green

- Blue

- White/Blue

- Green

- White/Brown

- Brown

Note:

T568B is the most commonly used standard in modern networks.

Straight-Through Cable

Definition

A straight-through cable uses the same wiring standard on both ends.

Example:

T568B on End A

and

T568B on End B

Uses

• Computer to Switch
• Computer to Router
• Switch to Router

Practical Session (30 Minutes)

Topic: Creating a Straight-Through Ethernet Cable

Objective

Students will create a straight-through Ethernet cable using the T568B wiring standard.

Requirements

• UTP Ethernet Cable
• RJ45 Connectors (2)
• RJ45 Crimping Tool
• Cable Tester

Procedure

Step 1: Measure and Cut Cable

• Measure the required cable length.
• Cut the cable using the crimping tool.

Step 2: Remove Outer Insulation

• Strip approximately 2 cm of the cable jacket.
• Expose the internal wire pairs.

Step 3: Untwist and Arrange Wires

Arrange wires according to T568B:

- White/Orange

- Orange

- White/Green

- Blue

- White/Blue

- Green

- White/Brown

- Brown

Step 4: Straighten the Wires

• Align all wires neatly.
• Ensure the order remains correct.

Step 5: Trim the Wires

• Cut all wires evenly.

Step 6: Insert into RJ45 Connector

• Carefully insert wires into the RJ45 connector.
• Verify that each wire reaches the end of the connector.

Step 7: Crimp the Connector

• Insert the connector into the crimping tool.
• Apply firm pressure to secure the connector.

Step 8: Repeat for the Other End

• Use the same T568B wiring sequence.
• Create the second connector.

Step 9: Test the Cable

• Connect both ends to a LAN cable tester.
• Verify that all eight wires are connected correctly.

Expected Result

• Cable tester shows correct pin-to-pin connectivity.
• No open or crossed connections.
• Straight-through cable is ready for network use.

Practical Task

Create a straight-through Ethernet cable and complete the following checklist:

□ Cable cut correctly

□ Outer insulation removed properly

□ T568B color sequence followed

□ RJ45 connectors attached correctly

□ Cable crimped successfully

□ Cable tester passed

Observation Sheet

- Cable Type: ___________

- Wiring Standard Used: ___________

- Number of RJ45 Connectors Used: ___________

- Cable Test Result: Pass / Fail

- Any Wiring Errors Found: ___________

Conclusion

Network cables are the backbone of wired communication. Understanding cable types, RJ45 connectors, and crimping techniques enables students to install and troubleshoot basic network infrastructure effectively.