# Day 4: Hands-on Subnetting

## 1. What is Subnetting?

Subnetting means:

Dividing one large IP network into smaller networks called subnets.

### Simple Example

Suppose a college has this network:

192.168.10.0/24

Instead of keeping all computers in one network, we can divide it into smaller networks.

For example:

College Network

192.168.10.0/24

|

+---- Computer Lab

|

+---- Faculty

|

+---- Administration

|

+---- Library

Each department can have its own subnet.

# 2. Why Do We Create Subnets?

Subnetting is useful for:

- Dividing a large network.

- Organizing devices.

- Separating departments.

- Reducing unnecessary broadcast traffic.

- Managing IP addresses.

- Improving network design.

- Making troubleshooting easier.

### Real-Life Example

Suppose a college has:

| Department | Required Devices |
| --- | --- |
| Computer Lab | 50 |
| Faculty | 20 |
| Administration | 15 |
| Library | 10 |

Instead of using one network for everyone, we can create separate subnets.

# 3. Basic Subnetting Terms

Students should know these terms before starting.

### Network Address

Identifies the subnet.

Example:

192.168.10.0

### First Usable IP

First IP that can normally be assigned to a host.

192.168.10.1

### Last Usable IP

Last IP that can normally be assigned to a host.

192.168.10.62

### Broadcast Address

Used to send traffic to all hosts in that subnet.

192.168.10.63

### Subnet Mask

Defines the network and host portions.

Example:

255.255.255.192

### CIDR

Short form of the subnet mask.

/26

# 4. Subnetting Formula Revision

For IPv4:

### Host bits

Host Bits = 32 - CIDR

### Total addresses

2^Host Bits

### Usable hosts

For a normal IPv4 subnet:

2^Host Bits - 2

### Number of subnets

If we borrow n bits:

2^n

# 5. Common Subnetting Table

Students should keep this table with them.

| CIDR | Subnet Mask | Total Addresses | Usable Hosts |
| --- | --- | --- | --- |
| /24 | 255.255.255.0 | 256 | 254 |
| /25 | 255.255.255.128 | 128 | 126 |
| /26 | 255.255.255.192 | 64 | 62 |
| /27 | 255.255.255.224 | 32 | 30 |
| /28 | 255.255.255.240 | 16 | 14 |
| /29 | 255.255.255.248 | 8 | 6 |
| /30 | 255.255.255.252 | 4 | 2 |

# 6. How to Create a Subnet Plan

Teach students this exact process.

## Step 1: Identify the Main Network

Example:

192.168.10.0/24

## Step 2: Find the Requirement

Suppose we need:

4 subnets

## Step 3: Find How Many Bits to Borrow

We need 4 subnets.

Formula:

2^n >= Required Subnets

For 4:

2^2 = 4

So we need to borrow:

2 bits

## Step 4: Calculate the New CIDR

Original:

/24

Borrow 2 bits:

24 + 2 = /26

New subnet:

/26

## Step 5: Find the Subnet Mask

/26 means:

255.255.255.192

## Step 6: Find Block Size

Use:

256 - 192 = 64

So the subnet increment is:

64

# 7. Create the Subnets

Starting network:

192.168.10.0/24

New subnet:

/26

Block size:

64

Therefore:

Subnet 1 → 192.168.10.0/26

Subnet 2 → 192.168.10.64/26

Subnet 3 → 192.168.10.128/26

Subnet 4 → 192.168.10.192/26

# 8. Complete Subnet Plan

| Subnet | Network | First Usable | Last Usable | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| 1 | 192.168.10.0/26 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 | 62 |
| 2 | 192.168.10.64/26 | 192.168.10.65 | 192.168.10.126 | 192.168.10.127 | 62 |
| 3 | 192.168.10.128/26 | 192.168.10.129 | 192.168.10.190 | 192.168.10.191 | 62 |
| 4 | 192.168.10.192/26 | 192.168.10.193 | 192.168.10.254 | 192.168.10.255 | 62 |

# 9. Assign Subnets to Departments

Now make the example more realistic.

| Department | Subnet |
| --- | --- |
| Computer Lab | 192.168.10.0/26 |
| Faculty | 192.168.10.64/26 |
| Administration | 192.168.10.128/26 |
| Library | 192.168.10.192/26 |

Now each department has its own subnet.

# 10. Assign IP Addresses to Devices

## Computer Lab

Network:

192.168.10.0/26

Possible device IPs:

PC-01 → 192.168.10.1

PC-02 → 192.168.10.2

PC-03 → 192.168.10.3

PC-04 → 192.168.10.4

Gateway, if a router is configured for this subnet, could be:

192.168.10.62

or another chosen usable address.

The important point is that the gateway address must be inside the same subnet and must be consistently configured on the router and hosts.

# 11. Faculty Subnet

Network:

192.168.10.64/26

Usable range:

192.168.10.65

to

192.168.10.126

Example:

Faculty-PC1 → 192.168.10.65

Faculty-PC2 → 192.168.10.66

Faculty-PC3 → 192.168.10.67

# 12. Administration Subnet

Network:

192.168.10.128/26

Usable range:

192.168.10.129

to

192.168.10.190

Example:

Admin-PC1 → 192.168.10.129

Admin-PC2 → 192.168.10.130

Admin-PC3 → 192.168.10.131

# 13. Library Subnet

Network:

192.168.10.192/26

Usable range:

192.168.10.193

to

192.168.10.254

Example:

Library-PC1 → 192.168.10.193

Library-PC2 → 192.168.10.194

Library-PC3 → 192.168.10.195

# 14. Important Practical Concept

Tell students:

Devices in the same subnet can communicate directly at Layer 2 when the local network is properly connected.

For example:

192.168.10.1

192.168.10.2

are in the same /26 subnet.

But:

192.168.10.1

192.168.10.65

belong to different /26 subnets.

Communication between different IP subnets normally requires a router or Layer 3 device.

# 15. Hands-on Practical: Create a Subnet

Now start the actual practical.

## Step 1

Give students:

Network:

192.168.20.0/24

Requirement:

Create 4 subnets.

Students calculate:

Required subnets = 4

2^2 = 4

Borrowed bits = 2

New CIDR = /26

Subnet Mask = 255.255.255.192

## Step 2

Students create:

Subnet 1:

192.168.20.0/26

Subnet 2:

192.168.20.64/26

Subnet 3:

192.168.20.128/26

Subnet 4:

192.168.20.192/26

# 16. Hands-on Practical: Assign IPs

If you have multiple computers connected to the same controlled lab network, assign addresses from the same subnet for the first connectivity exercise.

Example:

PC 1:

192.168.20.10

255.255.255.192

PC 2:

192.168.20.11

255.255.255.192

Then:

PC 1:

ipconfig

PC 2:

ipconfig

Verify the addresses.

Then from PC 1:

ping 192.168.20.11

If the network is correctly configured, the second PC should respond.

# 17. Important: Different Subnet Test

Now demonstrate:

PC 1:

192.168.20.10/26

and:

PC 2:

192.168.20.70/26

These belong to:

PC 1 → 192.168.20.0/26

PC 2 → 192.168.20.64/26

They are in different subnets.

Explain:

A switch alone does not route traffic between these IP subnets. A router or Layer 3 device is required.

This is an excellent way to make students understand why subnetting matters.

# 18. Verify IP Configuration

Students should use:

ipconfig

Check:

IPv4 Address

Subnet Mask

Default Gateway

For example:

IPv4 Address : 192.168.20.10

Subnet Mask  : 255.255.255.192

# 19. Test Local TCP/IP Stack

Before testing another computer, run:

ping 127.0.0.1

Expected:

Reply from 127.0.0.1

Explain:

127.0.0.1

↓

Local computer

# 20. Test Your Own IP

Run:

ipconfig

Suppose your IP is:

192.168.20.10

Then:

ping 192.168.20.10

This tests communication with your own IP address.

# 21. Test Another Computer

Suppose:

PC 1:

192.168.20.10

PC 2:

192.168.20.11

From PC 1:

ping 192.168.20.11

Students should record:

Reply

Request timed out

Destination host unreachable

Then discuss why the result occurred.

# 22. Troubleshooting Activity

If ping fails, ask students to follow this order:

### Check 1: IP Address

ipconfig

### Check 2: Subnet Mask

Check that both devices have the intended subnet mask.

### Check 3: Physical Connection

Check:

- Ethernet cable

- Switch port

- Wi-Fi connection

### Check 4: Ping Localhost

ping 127.0.0.1

### Check 5: Ping Own IP

ping <own-ip>

### Check 6: Ping Other PC

ping <other-pc-ip>

### Check 7: Firewall

A Windows firewall rule can block ICMP echo requests. Check the lab firewall configuration if the IP settings and physical connection are correct.

This gives students a practical troubleshooting flow instead of simply saying "ping is not working."

# 23. Subnet Plan Documentation

This is the main Day 4 Task.

Students should create a document containing the following sections.

## Section 1: Network Requirement

Example:

Main Network:

192.168.10.0/24

Required Subnets:

4

## Section 2: Calculate New CIDR

Required subnets = 4

2^2 = 4

Borrow 2 bits

Original CIDR = /24

New CIDR = /26

## Section 3: Subnet Mask

CIDR:

/26

Subnet Mask:

255.255.255.192

## Section 4: Subnet Table

Students create:

| Department | Network | First Usable | Last Usable | Broadcast | Hosts |
| --- | --- | --- | --- | --- | --- |
| Lab | 192.168.10.0/26 | 192.168.10.1 | 192.168.10.62 | 192.168.10.63 | 62 |
| Faculty | 192.168.10.64/26 | 192.168.10.65 | 192.168.10.126 | 192.168.10.127 | 62 |
| Admin | 192.168.10.128/26 | 192.168.10.129 | 192.168.10.190 | 192.168.10.191 | 62 |
| Library | 192.168.10.192/26 | 192.168.10.193 | 192.168.10.254 | 192.168.10.255 | 62 |

# 24. Device IP Assignment Table

Students should then create another table.

| Department | Device | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| Lab | PC-01 | 192.168.10.1 | 255.255.255.192 |
| Lab | PC-02 | 192.168.10.2 | 255.255.255.192 |
| Faculty | PC-01 | 192.168.10.65 | 255.255.255.192 |
| Faculty | PC-02 | 192.168.10.66 | 255.255.255.192 |
| Admin | PC-01 | 192.168.10.129 | 255.255.255.192 |
| Library | PC-01 | 192.168.10.193 | 255.255.255.192 |

# 25. Network Diagram Activity

Ask students to draw:

Router

|

+----------+----------+

|          |          |

SW1        SW2        SW3

|          |          |

Lab       Faculty      Admin

|

PCs

For a more realistic design, students can show the subnet next to each department:

Lab

192.168.10.0/26

Faculty

192.168.10.64/26

Admin

192.168.10.128/26

Library

192.168.10.192/26

Explain that a real network would need appropriate switch/router configuration for communication between different subnets.

# 26. Practical Exercise 2 – Different Number of Subnets

Give students:

Network:

192.168.50.0/24

Requirement:

Create 8 subnets.

Students should calculate:

2^3 = 8

Borrow 3 bits

New CIDR:

/27

Subnet Mask:

255.255.255.224

Block size:

256 - 224 = 32

Subnets:

192.168.50.0/27

192.168.50.32/27

192.168.50.64/27

192.168.50.96/27

192.168.50.128/27

192.168.50.160/27

192.168.50.192/27

192.168.50.224/27

Each subnet:

32 total addresses

30 usable hosts

# 27. Practical Exercise 3 – Requirement Based Subnetting

Give students:

Network:

192.168.100.0/24

Requirements:

Lab       → 50 hosts

Faculty   → 20 hosts

Office    → 10 hosts

Ask:

Which subnet size should be used for each department?

Students should understand that using the same /26 for every department works, but it wastes addresses for smaller departments.

This introduces the concept of VLSM (Variable Length Subnet Masking).

For this Day 4, you can keep VLSM as an introduction and do detailed VLSM in a later class.

# 28. Day 4 Student Task

Give students this task without the solution.

## Task: Create a Subnet Plan

Network:

192.168.40.0/24

Requirement:

Create 4 departments:

1. Computer Lab

2. Faculty

3. Administration

4. Library

Students must prepare:

### Part 1

Find:

New CIDR

Subnet Mask

Block Size

### Part 2

Calculate for all four subnets:

Network Address

First Usable IP

Last Usable IP

Broadcast Address

Total Addresses

Usable Hosts

### Part 3

Assign at least:

3 devices per department

### Part 4

Create a network diagram.

### Part 5

Write the commands used to verify the configuration:

ipconfig

ping <destination-ip>

# 29. Day 4 Final Practical Workflow

Students should follow this exact sequence:

Main Network

↓

Identify Requirement

↓

Calculate Required Subnets

↓

Borrow Host Bits

↓

Find New CIDR

↓

Find Subnet Mask

↓

Calculate Block Size

↓

Create Subnet Ranges

↓

Find Network Address

↓

Find First Usable

↓

Find Last Usable

↓

Find Broadcast

↓

Assign IP Addresses

↓

Configure Devices

↓

Verify with ipconfig

↓

Test with ping

↓

Document the Subnet Plan

# Day 4 Expected Learning

By the end of Day 4, students should be able to:

- Explain subnetting in simple words.

- Understand why subnetting is required.

- Divide a /24 network into smaller subnets.

- Calculate the required CIDR.

- Calculate subnet masks.

- Calculate block size.

- Find network addresses.

- Find first and last usable IPs.

- Find broadcast addresses.

- Calculate total and usable addresses.

- Assign IP addresses to devices.

- Verify IP configuration using ipconfig.

- Test connectivity using ping.

- Understand same-subnet vs different-subnet communication.

- Create a basic network diagram.

- Prepare and document a subnet plan.

### Most important practical formula for Day 4

Required Subnets

↓

2^n ≥ Required Subnets

↓

Borrow n bits

↓

New CIDR = Old CIDR + n

↓

Find Subnet Mask

↓

Find Block Size

↓

Create Subnet Ranges

↓

Assign IPs

↓

Test Connectivity

This makes Day 4 a proper hands-on subnetting lab, rather than repeating the theory from Day 3.